NetWare Driver Installation 		 Dec. 1994

===========================

Copyright Notice:  First Edition, April 1994

Copyright Digital Equipment Corporation 1994.

All rights reserved.

Copyright Novell, Inc. 1994.

NetWare is a registered trademark of Novell Inc.

Alpha AXP, AXP, DEC, DECchip, DECnet, Digital, PATHWORKS,

ThinWire, and the DIGITAL logo, are  trademarks of Digital

Equipment Corporation.

All other trademarks and registered trademarks are the property

of their respective holders.

Do not redistribute the *.NLM files without a written permission

from Novell.

The NetWare driver in this diskette is organized as follows:

Directory	      Contents

-----------------     --------------------------------------------------------

\CLIENT		      ODI DOS driver program for Novell Wks (3.11,3.12,4.0,4.1)

\SERVER		      Support NetWare 3.11, 3.12 or 4.x server

DC21040  LAN       DE-530CT server driver

\SERVER\NW311

MSM31X   NLM       Supports Novell NetWare ODI drivers on 3.11 server

ETHERTSM NLM       Support Novell NetWare ODI drivers on 3.11 server

PATCHMAN NLM       Support Novell NetWare ODI drivers on 3.11 server

AUTOEXEC NCF       Addition to your server AUTOEXEC.NCF

LSLENH   NLM       Support Novell NetWare ODI drivers on 3.11 server

MONITOR  NLM       Support Novell NetWare ODI drivers on 3.11 server

\SERVER\NW312

MSM	    NLM       Supports Novell NetWare ODI drivers on 3.12 server

ETHERTSM NLM       Support Novell NetWare ODI drivers on 3.12 server

AUTOEXEC NCF       Addition to your server AUTOEXEC.NCF

\SERVER\NW40X

MSM	    NLM       Support Novell NetWare ODI drivers on 4.x server

ETHERTSM NLM       Support Novell NetWare ODI drivers on 4.x server

AUTOEXEC NCF       Addition to your server AUTOEXEC.NCF

DC21040  LDI       Reference file for installation

***********************

*  WORKSTATION SETUP  *

***********************

1)  Modify the NET.CFG file to reflect your network environment/setup.

2)  Afterwards, you can run the LSL, DC21040, IPXODI, and NETX programs

and connect to a Novell server:

>LSL       --This program can be found in this diskette \NETWARE\CLIENT

>DC21040

>IPXODI

>NETX      --This program is provided by Novell

>F:

>LOGIN username  --Login to Novell server

******************

*  SERVER SETUP  *

******************

New Installation of NetWare 3.11 Server

=======================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled SYSTEM-1.

>Copy A:\NETWARE\SERVER\*.*       B:\   --SYSTEM-1 disk must be in drive B

>Copy A:\NETWARE\SERVER\NW311\*.*  B:\

2)  Execute Novell's SERVER program, which resides in the SYSTEM-1 diskette.

After loading the hard disk driver, you can load the network driver as

follows:

:Load Patchman		--load these four NLM files before

:Load Lslenh 		  loading the DC21040.LAN driver

:Load Msm31x

:Load Ethertsm

:Load B:\DC21040.LAN  TP	--if you are using twisted pair cable

or

:Load B:\DC21040.LAN  AUI	--if you are using coax cable

:Bind ipx to DC21040  net=1

:load install		--Follow the NetWare instructions and finish

the installation.

3)  Your AUTOEXEC.NCF should contain the following network drivers:

Load Patchman

Load Lslenh

Load Msm31x

Load Ethertsm

Load DC21040.LAN  TP   or   AUI

Bind Ipx to DC21040  net=1

Modifying an Existing NetWare 3.11 Server

=========================================

1)  Login to the Novell server as a SUPERVISOR.

2)  Copy all supplied NLM and LAN files to your server's SYSTEM directory:

>Copy A:\NETWARE\SERVER\NW311\*.*   F:\SYSTEM

>Copy A:\NETWARE\SERVER\*.LAN      F:\SYSTEM

3)  Modify the server's AUTOEXEC.NCF to follow the loading sequences described

in the \NETWARE\SERVER\NW311\AUTOEXEC.NCF file of this diskette.

Load Patchman

Load Lslenh

Load Msm31x

Load Ethertsm

Load DC21040.LAN  TP   or   AUI

Bind Ipx to DC21040  net=1

4)  Shutdown the Server and then turn it ON again.  This time, the DE530CT

will be installed as your network card.

5)  Note that you may override the default configuration cable selection by

typing one of the following keywords in the LOAD line: AUI or TP:

load DC21040 frame=ethernet_ii AUI	 --if you are using coax cable

or

load DC21040 frame=ethernet_ii TP	 --if you are using twisted pair cable

Modifying an Existing NetWare 3.12 Server

=========================================

1)  Login to the Novell server as a SUPERVISOR.

2)  Copy all supplied NLM and LAN files to your server's SYSTEM directory:

>Copy A:\NETWARE\SERVER\NW312\*.*   F:\SYSTEM

>Copy A:\NETWARE\SERVER\*.LAN      F:\SYSTEM

3)  Modify the server's AUTOEXEC.NCF to follow the loading sequences described

in the \NETWARE\SERVER\NW312\AUTOEXEC.NCF file of this diskette:

load c:\NETWARE\MSM.NLM

load c:\NETWARE\ETHERTSM.NLM

load c:\NETWARE\DC21040.LAN (TP or AUI)

bind ipx to DC21040 NET=number

load DC21040 frame=ethernet_II  AUI or TP

4)  Shutdown the Server and then turn it ON again.  This time, the DE530CT

will be installed as your network card.

5)  Note that you may override the default configuration cable selection by

typing one of the following keywords in the LOAD line: AUI or TP:

load DC21040 frame=ethernet_ii AUI	 --if you are using coax cable

or

load DC21040 frame=ethernet_ii TP	 --if you are using twisted pair cable

Modifying an Existing NetWare 4.x Server

========================================

1)  Login to the Novell server as a SUPERVISOR.

2)  Copy all supplied NLM and LAN files to your server's SYSTEM directory:

>Copy A:\NETWARE\SERVER\NW40X\*.*   F:\SYSTEM

>Copy A:\NETWARE\SERVER\*.LAN     F:\SYSTEM

3)  Modify the server's AUTOEXEC.NCF to follow the loading sequences described

in the \NETWARE\SERVER\NW40X\AUTOEXEC.NCF file of this diskette:

load c:\NETWARE\MSM.NLM

load c:\NETWARE\ETHERTSM.NLM

load c:\NETWARE\DC21040.LAN (TP or AUI)

bind ipx to DC21040 NET=number

load DC21040 frame=ethernet_II  AUI or TP

4)  Shutdown the Server and then turn it ON again.  This time, the DE530CT

will be installed as your network card.

5)  Note that you may override the default configuration cable selection by

typing one of the following keywords in the LOAD line: AUI or TP:

load DC21040 frame=ethernet_ii AUI	 --if you are using coax cable

or

load DC21040 frame=ethernet_ii TP	 --if you are using twisted pair cable