Dec. 1994

Windows NT v3.5 Driver Installation

for

D-Link DE-530 PCMCIA Ethernet Adapter

=======================================

The DE-530CT PCI Ethernet LAN drivers of DE-530CT cards for Windows NT

contained in this diskette are as follows:

Files		     Contents

--------------	     ----------------------------------------------------

DC21040.SYS	     NDIS 3.0 driver for Windows NT.

OEMSETUP.INF	     OEMSETUP file for Windows NT.

\DISK1

**********************************

*  Windows NT V3.5 Installation  *

**********************************

Installation Procedure:

(1)  When LAN drivers is installed, Windows NT will present a list of all

available drivers.  Since there will be no D-Link DE-530 drivers initially,

select OTHER for 3rd party adapters not in the driver list.

(2)  Windows NT will ask for the drive/path containing the DE-530 Windows NT

drivers.  Insert the D-Link driver diskette in drive A: and respond with

the following.

A:\NT35

(3)  Windows NT will attempt to locate an OEMSETEP.INF file in the specified

path. If you have entered the path name correctly, Windows NT should copy

the appropriate drivers to the Windows NT system.

(4)  At this point, the driver would have been properly copied and the

installation would proceed to the next step.  Please refer to your Windows

NT installation guide for correct procedures.

(5)  After the initial installation, the DE-530 drivers will be permanently

copied to the Windows NT system.  Subsequent network driver lists will

already contain D-Link DE-530 driver.