NetWare Driver Installation             November 1992

===========================

The NetWare driver programs in this disk support the following D-Link cards:

DE-600                      DE-600-E

DE-600TP                    DE-600TP-E

DE-600AUI                   DE-600AUI-E

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Directory             Contents

---------             --------------------------------------------------------

\NETWARE              A pre-generated IPXDE600.COM

\NETWARE\V215B        NetWare v2.15b workstation drivers

\NETWARE\V215C        NetWare v2.15c, v2.2 or NetWare 386 workstation drivers

\NETWARE\ODI.DOS      ODI workstation driver for DOS

\NETWARE\ODI.OS2      ODI workstation driver for OS/2

*********************************

*  QUICK SETUP FOR WORKSTATION  *

*********************************

A pre-generated workstation driver IPXDE600.COM is included in this disk.

You can simply run the IPXDE600 on a workstation to access the Novell network:

>IPXDE600    --Supports NetWare Lite, 2.1x, 2.2 or NetWare 386

>NETx        --This program is provided by Novell

***********************

*  WORKSTATION SETUP  *

***********************

If you want to generate your own workstation driver instead of using the

provided IPXDE600.COM, please do the following:

1)  If you make a backup of this diskette, make sure that it is labeled as

LAN_DRV_DLK.  You can use the DOS "Label" command as follows:

>Label  A:LAN_DRV_DLK

2)  Copy the appropriate NetWare driver files to the root directory of

this diskette (or backup):

>Copy  A:\NETWARE\V215c\*.*  A:\   --For NetWare 386 workstation

3)  Execute Novell's SHGEN or WSGEN program, which resides in the SHGEN-1 or

WSGEN diskette.  When you are asked to select a network driver, press

the INS key.  You will be asked to insert a LAN_DRV_??? diskette.  Insert

this diskette (or backup) in drive A.  Upon completion, a new IPX.COM file

will be created in your SHGEN-2 or WSGEN diskette.

************************

*  NETWARE LITE USERS  *

************************

1)  Follow the normal installation procedures as instructed by NetWare Lite.

When asked to select a network driver, simply choose "NOVELL NE2000".

2)  After installation completed, modify the STARTNET.BAT file as follows:

From:

---------------------------

LSL

NE2000

IPXODI A

CLIENT

To:

---------------------------

LSL

DE600ODI

IPXODI A

CLIENT

3) Copy \NETWARE\ODI.DOS\NET.CFG file to NWLITE sub-directory which was

created when you installed NetWare Lite.

4) You may run STARTNET to access the Novell network.

************************************

* Install NetWare V2.2 workstation *

************************************

Refer to procedure  A. HOW TO SETUP IPX and follow all procedures

except for #3.  Instead of running the program SHGEN.EXE, run WSGEN.EXE .

**********************

*  ODI DRIVER SETUP  *

**********************

ODI Workstation for DOS

=======================

D-Link ODI Driver program allows you to run multiple protocl stacks.  It also

allows you to install up to two network adapters in a workstation. D-Link ODI

driver supports "port, node address, protocol and frame" options.  You can use

these options to create your own NET.CFG file.  For detailed info on Keywords,

please refer to Novell manual "NetWare ODI shell for DOS - NET.CFG options".

1)  Copy the DE600ODI.COM file (from this diskette) to the directory which

contains Novell's LSL.COM, IPXODI.COM and NETx.COM files.

2)  Follow Novell's instructions on using the ODI programs. You can substitute

the DE600ODI.COM whenever a reference is made to the Hardware Specific Module.

3)  A typical startup for ODI workstation is as follows:

>LSL               --Link Support Layer Module provided by Novell

>DE600ODI          --Hardware Specific Module provided by D-Link

>IPXODI            --IPX Protocol Stack Module provided by Novell

>NETx              --DOS Support Module provided by Novell

>LOGIN

ODI Workstation for OS/2

========================

1)  You must first install the Requester on the workstation.  For detailed

information, refer to Novell manual "NetWare Requester for OS/2".

2)  Copy DE600ODI.SYS (from this diskette) to your C:\NETWARE directory.

Afterwards, add the following device driver statement to your CONFIG.SYS

file:

DEVICE=C:\NETWARE\DE600ODI.SYS

3)  D-Link ODI driver supports "port, node address, protocol and frame"

options.  You can use these options to create your own NET.CFG file. For

detailed information on Keywords, please refer to Novell manual

"NetWare Requester for OS/2 - Reconfiguring the NetWare Requester".