November 1992

PC-NFS Driver Installation

for

D-Link Ethernet Pocket LAN Adapter

=========================================

The D-Link PC-NFS Driver Program lets you run Sun MicroSystems PC-NFS

on your PC with the D-Link Ethernet Pocket LAN Adapter.

The following files will be used in this installation:

Files           Contents

-----------     ----------------------------------------------------------

PDNFS.SYS       PC-NFS Driver Program

DE600PD.SYS     Packet driver program for DE-600 card

DE600PD.COM     Packet driver program for DE-600 card

Two methods can be used to install:

METHOD 1:

1. Install the PC-NFS program according to the procedure described in the

PC-NFS manual.  On the screen of the PC-NFS install program, select 3C501

(the 3Com EtherLink card) as your network interface card. When the screens

prompt you, select 3 for the interrupt level (IRQ), and select 300h for

the base I/O address.

2. Copy the following files to the \NFS directory of your hard

disk:

PDNFS.SYS

DE600PD.SYS

DE600PD.COM

3. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE =C:\NFS\DE600PD.SYS <packet_int_no>

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

SET NFSDRIVE = C

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

4. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate

other computers through the TCP/IP protocol.

METHOD 2:

1. Install the PC-NFS program according to the procedure described in the

PC-NFS manual.  On the screen of the PC-NFS install program, select 3C501

(the 3Com EtherLink card) as your network interface card. When the screens

prompt you, select 3 for the interrupt level (IRQ), and select 300h for

the base I/O address.

2. Copy the following files to the \NFS directory of your hard

disk:

PDNFS.SYS

DE600PD.SYS

DE600PD.COM

3. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

SET NFSDRIVE = C

DE600PD <packet_int_no>

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

4. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate

other computers through the TCP/IP protocol.