MS Windows for Workgroups Installation With D-Link Cards

========================================================

This driver diskette includes an OEMSETUP.INF file for Windows for Workgroups.

Using the OEMSETUP.INF file, the user can select whichever D-Link network

adapters are to be installed based on Windows dialog menus.

The OEMSETUP.INF file contains selections for all the following D-Link

network adapters:

DE-100 series, DE-200 series, DE-205TP, DE-220 series

DE-300 series, DE-320 series, DE-400, DE-600 series

DE-620 series

Installation procedures:

1. Proceed to install Windows for Workgroups.  If you already have an

installed Windows for Workgroups, open the Control Panel Window to

change the network settings.

2. When Windows asks for the type of network adapter, it will show a

list of all network adapters supported by Windows.  You must select

'Unlisted or Updated Network Adapter'.

3. Windows will ask for an OEMSETUP.INF file. Since the D-Link OEMSETUP.INF

file is included in the D-Link driver diskette, insert the driver diskette

in drive A:

Input the full path name where the D-Link OEMSETUP.INF file is located.

(i.e. A:\NDIS\OEMSETUP.INF)

4. You will see a Windows dialog menu listing all D-Link network adapters

included in the OEMSETUP.INF file.  Select the network adapter that

you will be using.

5. After selecting the type of D-Link network adapter, Windows will look

for the NDIS driver of the network adapter you have selected.

At this time, the D-Link driver diskette should still be in drive A:.

Input the full path name where the D-Link NDIS driver is located.

(i.e. A:\NDIS\DOS\)

6. Windows will copy the NDIS driver file and update the Windows network

system automatically.  Be sure to configure the network adapter if you

are using I/O port or IRQ other than the default.

At this point, you can finish the installion or continue the process

according to your preferences.

7. After installation is completely finished, restart your system for the

updates to take effect.

D-Link NDIS PROTOCOL.INI User Guide

===================================

This guide lists the available PROTOCOL.INI keywords for all

D-Link NDIS device drivers.  Most of the keywords are optional

and have default values.  The asterisk symbol (*) indicates the

list of D-Link adapters where the keyword is applicable.

IOADDRESS - This keyword specifies the base I/O port address used

by the D-Link Network Interface Card. This value must

match the base I/O address jumper setting on your NIC.

If this keyword is not present, the default value will

be 300h.

*all D-Link network adapters

INTERRUPT - This keyword specifies the interrupt line used by the

D-Link NIC. This value must match the IRQ jumper setting

on your NIC.  If this keyword is not present, the default

value will be 3.

*all D-Link network adapters

DRIVERNAME- This keyword specifies the NIC driver's name.  The NIC

driver name follows the NIC adapter type.  (i.e. DE100,

DE200, DE220, etc.)

*all D-Link network adapters

PRINTPORT - This keyword specifies the printer port used by your

D-Link Pocket LAN Adapter.

*DE-600, DE-620

BASEMEMORY- This keyword specifies the mapped memory address used

by the D-Link NIC.  This value must match the base

memory jumper setting on your NIC.  Base memory will

only be in effect if the remote boot ROM jumper is

enabled.

*DE-100, DE-200

ADAPTERID - This keyword specifies the adapter ID used by D-Link

MCA bus card. This value must match the card ID jumper

setting on your MCA adapter.

*DE-300, DE-320

NETADDRESS- This keyword specifies the node address (hex) used by

your network interface card.  This option is used only

if you want to override the node address burned in your

NIC.  In this case, the specified address will be used

instead of the NIC's actual node address.

*all D-Link network adapters