�����������������������������������������ͻ

�                                         �

�      DL3520 TECHNICAL SPECIFICATION     �

�                                         �

�����������������������������������������ͼ

Document Number : DE-000-000

Revision        : 1.00

Modify Date     : JUN 10,1992

Released Date   : SEP 30,1992

D-LINK CORPORATION.

No.20, Park Ave. II

Science-Based Industrial Park.

Hsinchu, Taiwan

Republic of China

Tel: 886-35-779966

Fax: 886-35-777154

D-LINK CONFIDENTIAL

DL3520  SPECIFICATION

=====================

SCOPE :

The function and performance of DL3511 lan controller are no more

leading the pocket LAN market day after days. It is necessary to

have new generations of pocket Ethernet LAN controller to replace

DL3511. The update version pocket LAN controller design plan is

now completed named DL3520.

DL3520 is a high performance pocket LAN controller that reduce

instruction set, direct reporting status, and support all kinds of

printer port data mode includes uni-direction, bi-direction and EPP

modes.

FEATURES :

1. Supports IEEE 802.3/ANSI 8802-3 and Ethernet standard.

2. Printer port interface is integrated.

3. Support uni-direction, bi-direction and EPP printer modes.

4. Implements Printer interface handshake function .

5. Printer interface has data FIFO .

6. Command or data debounding delay factors are software settable.

7. Has EEPROM interface to access node ID and other information.

8. Transmit buffer is single in simple mode and can be dual buffers

if necessary.

9. Support auto-transmit mode to transmit packets.

10. Automatic transmit byte count assignment or padding.

11. Receive buffer ring is implemented.

12. Buffer RAM are selectable 8K, 16K, 32K or 64K bytes.

13. Four LED indicators.

Grean  : Power / Txm        Yellow : collision

Grean  : Link  / Rcv        Yellow : Jabber

14. Implemented in low power CMOS technology.

15. 80 pins mini size PQFP package.

P.1                  D-LINK CONFIDENTIAL

DL3520 APPLICATION PRODUCTS SPECIFICATION :

1.  10 base-2/T , two media in one model and supports single type.

@ DE-620CT is for 10BASE2 and 10BASE-T

@ DE-620CX is for 10BASE2

@ DE-620TP is for 10BASE-T

@ DE-620AUI is for 10BASE5

2.  Uses traditional AC adapter with UL/DUV safety.

output voltage is 12V DC. 500mA for all models.

3.  Uses two sides SMT production technology.

4.  Supports fixed Duplex-port (printer port) function and

the power of Duplex-port is supported by DE620.

P.2                  D-LINK CONFIDENTIAL

DL3520 BLOCK DIAGRAM :

D  S  D  C

O  K  I  S

�  �  �  �

������������Ŀ      ������������Ŀ        �������Ŀ

��Ŀ           �DATA /      �      �   EIP      �        �       �

�S �           �STATUS      �      ������������Ĵ        �  MA   �

�TP�<���������͵MULTIPLEXER �      �            ��������>�  BUS  �

�AO�   �������>�   &        �      �            �        � DRIVER�

�TR�   �       �BYTE/NIBBLE �<����͵            �        ���������

�UT�   � �����>�TRANSLATER  �   �  � INTERNAL   �

�S �   � �     ��������������   �  � REGISTERS  �        �������Ŀ

����   � �                      �  �            �        �       �

�ͳͺ���������������������ͼ  �            �        �  MD   �

� � �                         �            �<������>�  BUS  �

� � �     ���������Ŀ         �     &      �    �   � BUFFER�

��Ŀ � � �     � DATA    �         �            �    �   ���������

�  � � � �����>� BUFFER  ���������>� WORKING    �    �

�DP� � � �����>�         �         � REGISTERS  �    �

�AO�<� � ��    �����������   �����>�            �    �

�TR�<���͹�    ���������Ŀ   �     �            �    �

�AT�   � ��    � COMMAND ���ͼ     ��������������    �   �������Ŀ

�  �   � �����>� BUFFER  �         ������������Ŀ    ���>�       �

����   �  � ��>�    &    �<�������ĳ            �<��ĺ�� �       �

�  � �  � DECODER ���������>�            ����ĺ��>�  TXM  �

�  � �  �����������         �            �    �   �       �

�  � ��������������������Ŀ �            �    �   ���������

��Ŀ   �  ��������������������Ŀ � �            �    �

�C �   ���������������������Ŀ � � �   LOCAL    �    �

�OP�          ������������Ŀ � � � �    BUS     �    �

�MO�          � DEBOUNDING � � � � � ARBITRATOR �    �   �������Ŀ

�MR�          �     &      ��� � � �            �    �   �       �

�AT����������>�  FALLING   ����� � �            �    ���͵       �

�N �          �   EDGE     ������� �            �<������ �  RCV  �

�D �          �  DETECTOR  �       �            ��������>�       �

����          ��������������       ��������������        ���������

P.3                  D-LINK CONFIDENTIAL

INTERNAL REGISTERS AND ABBREVIATION :

������������������������������������������������������Ŀ

�                  WRITABLE REGISTERS                  �

������������������������������������������������������Ĵ

�  COM    � Command register                           �

������������������������������������������������������Ĵ

�  TBR1   � Transmit byte count register 1 (High byte) �

������������������������������������������������������Ĵ

�  TBR0   � Transmit byte count register 0 (Low  byte) �

������������������������������������������������������Ĵ

�  RSA1   � Remote-read start address 1    (High byte) �

������������������������������������������������������Ĵ

�  RSA0   � Remote-read start address 0    (Low  byte) �

������������������������������������������������������Ĵ

�  NPR    � Next page register                         �

������������������������������������������������������Ĵ

�  DFR    � Delay factor register ( WR or RD )         �

������������������������������������������������������Ĵ

�  CPR    � Current page register                      �

������������������������������������������������������Ĵ

�  SPR    � Start page register                        �

������������������������������������������������������Ĵ

�  EPR    � End page register                          �

������������������������������������������������������Ĵ

�  SCR    � System configuration register              �

������������������������������������������������������Ĵ

�  TCR    � Transceiver configuration register         �

������������������������������������������������������Ĵ

�  EIP    � EEPROM interface port                      �

������������������������������������������������������Ĵ

�  PAR0   � Physical address register 0                �

������������������������������������������������������Ĵ

�  PAR1   � Physical address register 1                �

������������������������������������������������������Ĵ

�  PAR2   � Physical address register 2                �

������������������������������������������������������Ĵ

�  PAR3   � Physical address register 3                �

������������������������������������������������������Ĵ

�  PAR4   � Physical address register 4                �

������������������������������������������������������Ĵ

�  PAR5   � Physical address register 5                �

��������������������������������������������������������

������������������������������������������������������Ŀ

�                  READABLE REGISTERS                  �

������������������������������������������������������Ĵ

�  STS    � Status register                            �

������������������������������������������������������Ĵ

�  CPR    � Current page register                      �

������������������������������������������������������Ĵ

�  BPR    � Boundary page register ( = NPR - 1 normal )�

������������������������������������������������������Ĵ

�  TDR    � Time domain reflectometry register         �

��������������������������������������������������������

P.4                  D-LINK CONFIDENTIAL

DL3520  INTERFACES ARE DEVIDED INTO FIVE PARTS :

1. Buffer RAM interface.

2. Network interface

3. Printer port interface.

4. EEPROM interface.

5. Power and system used signals.

Pin descriptions :

1. Buffer RAM interface

��������������������������������������������������������������������������Ŀ

� Symbol � Pin No � Function �               Description                   �

��������������������������������������������������������������������������Ĵ

� MA0 -  � 59,58..� I/O      �Memory address bus :                         �

�  MA6,  � .46,45 �          � These signals are used to addressing to     �

� MA7 -  �        � O        � local RAM buffer up to 32K bytes. Total RAM �

�  MA14  �(61,60..�          � buffer size can be expanded to 64K bytes.   �

�        � .48,47)�          � See below discription.                      �

�        �*Note1  �          �                                             �

��������������������������������������������������������������������������Ĵ

� /MCSL  �  39    � O/I      �Memory chip select low or inverter input :   �

� or     �        �          � When MA4 is pulled low, /MCSL is selected as�

� INVIN  � (41)   �          � output to select lower 8K/32K RAM buffer.   �

�        �        �          � Otherwise the pin is reserved as inverter   �

�        �        �          � input. See appendix A.                      �

��������������������������������������������������������������������������Ĵ

� /MCSH  �  38    � O        �Memory chip select high or inverter output : �

� or     �        �          � When MA4 is pulled low, /MCSH is selected as�

� INVOUT � (40)   �          � output to select upper 8K/32K RAM buffer.   �

�        �        �          � Otherwise the pin is reserved as inverter   �

�        �        �          � output.                                     �

��������������������������������������������������������������������������Ĵ

� MD0 -  � 37,36..� I/O      �Memory data :                                �

�  MD7   � .31,30 �          � The memory data bus is connected to local   �

�        �(39,38..�          � RAM buffer and is used to access RAM data.  �

�        � .33,32)�          �                                             �

��������������������������������������������������������������������������Ĵ

� /MR    �  41    � O        �Memory read :                                �

�        �        �          � This signal indicates a read operation to   �

�        � (43)   �          � the local RAM buffer. It is active low.     �

��������������������������������������������������������������������������Ĵ

� /MW    �  40    � O        �Memory write :                               �

�        �        �          � This signal indicates a write operation to  �

�        � (42)   �          � the local RAM buffer. Data should be latched�

�        �        �          � into RAM at rising edge of the signal.      �

����������������������������������������������������������������������������

P.5                  D-LINK CONFIDENTIAL

2. Network interface

��������������������������������������������������������������������������Ŀ

� Symbol � Pin No � Function �               Description                   �

��������������������������������������������������������������������������Ĵ

� COL    �  29    �  I       �Collision detect:                            �

�        �        �          � This signal high indicates a collision has  �

�        � (31)   �          � been detected on the network. It also       �

�        �        �          � indicates the heartbeat condition at the end�

�        �        �          � of each transmition.                        �

��������������������������������������������������������������������������Ĵ

� RXD    �  28    �  I       �Receive data :                               �

�        � (30)   �          � Serial data received from ENDEC             �

��������������������������������������������������������������������������Ĵ

� CSN    �  27    �  I       �Carrier Sense :                              �

�        �        �          � This signal  indicates  the  presence  of   �

�        � (29)   �          � carrier on the network. It is provided by   �

�        �        �          � ENDEC and is active high.                   �

��������������������������������������������������������������������������Ĵ

� RXC    �  26    �  I       �Receive clock :                              �

�        �        �          � A clock provided by ENDEC to clock receiving�

�        � (28)   �          � data  into  DL3520 . Additional information �

�        �        �          � see appendix A 'NS' partion.                �

��������������������������������������������������������������������������Ĵ

� TXD    �  24    �  O       �Transmit data :                              �

�        �        �          � Serial data output to the ENDEC. The data   �

�        � (26)   �          � is valid on the rising edge of TXC.         �

��������������������������������������������������������������������������Ĵ

� TXC    �  21    �  I       �Transmit clock :                             �

�        �        �          � The clock is used to provide timing for     �

�        � (23)   �          � shifting data out of the DL3520 . It is     �

�        �        �          � normally provided by the ENDEC and is a 10  �

�        �        �          � MHz clock.                                  �

��������������������������������������������������������������������������Ĵ

� TXE    �  20    �  O       �Transmit enable :                            �

�        �        �          � This signal becomes active high when the    �

�        � (22)   �          � first transmitting bit is available. It     �

�        �        �          � remains active until the last bit is clocked�

�        �        �          � out.                                        �

��������������������������������������������������������������������������Ĵ

� LBK    �  25    �  O       �Loopback :                                   �

�        �        �          � This signal is active high to inform NS8391 �

�        � (27)   �          � type ENDEC to perform loopback operation.   �

����������������������������������������������������������������������������

P.6                  D-LINK CONFIDENTIAL

3. Printer port interface

��������������������������������������������������������������������������Ŀ

� Symbol � Pin No � Function �               Description                   �

��������������������������������������������������������������������������Ĵ

� CD0 -  � 76 - 80� I/O      �Command or data bus :                        �

�  CD7   � ,1,4,5 �          � The bus refer to printer port DATA0-DATA7.  �

�        �        �          � It is bidirectional. When it serve as a     �

�        �(78-80, �          � input,host can write command or data through�

�        � 1-3,6, �          � this port into DL3520  with a command strobe�

�        � 7     )�          � or data strobe. When it serve as a output,  �

�        �        �          � host can read data from this port.          �

��������������������������������������������������������������������������Ĵ

� RDY    �  73    � O        �Command ready indicater:                     �

� *Note2 �        �          �ICEN active :  *Note3                        �

�        � (75)   �          � When RDY is high it indicates DL3520  ready �

�        �        �          � to accept next command or data. It is hand- �

�        �        �          � shake signal between DL3520  and printer    �

�        �        �          � port.                                       �

�        �        �          �ICEN inactive :                              �

�        �        �          � The pin is in tri-state.                    �

��������������������������������������������������������������������������Ĵ

� D0/D4  �  71    � O        �ICEN active :                                �

� or     �        �          � When nibble mode is selected (default),and  �

� TXBZ0  � (73)   �          � during the read command period. The pin out-�

�        �        �          � put data bit0 or bit4, high bit (nibble)    �

�        �        �          � first and then low bit. All other condition �

�        �        �          � the pin will output TXBZ0 ( transmit buffer �

�        �        �          � 0 busy ) status. When remote write 0 command�

�        �        �          � is initiated,TXBZ0 will go high untill TXM  �

�        �        �          � is completed or TXM attempt equal 16.       �

�        �        �          �ICEN inactive :                              �

�        �        �          � The pin is in tri-state.                    �

��������������������������������������������������������������������������Ĵ

� D1/D5  �  72    � O        �ICEN active :                                �

� or     �        �          � When nibble mode is selected (default),and  �

� TXBZ1  � (74)   �          � during the read command period. The pin out-�

�        �        �          � put data bit1 or bit5, high bit (nibble)    �

�        �        �          � first and then low bit. All other condition �

�        �        �          � the pin will output TXBZ1 status.           �

�        �        �          �ICEN inactive :                              �

�        �        �          � The pin is in tri-state.                    �

��������������������������������������������������������������������������Ĵ

� D2/D6  �  69    � O        �ICEN active :                                �

� or INT �        �          � When nibble mode is selected (default),and  �

�        � (71)   �          � during the read command period. The pin out-�

�        �        �          � put data bit2 or bit6, high bit (nibble)    �

�        �        �          � first and then low bit. All other condition �

�        �        �          � the pin will output INTerrupt signal.       �

�        �        �          �ICEN inactive                                �

�        �        �          � When /INTT low the pin is in tri-state.     �

����������������������������������������������������������������������������

P.7                  D-LINK CONFIDENTIAL

3. Printer port interface ( continue )

��������������������������������������������������������������������������Ŀ

� Symbol � Pin No � Function �               Description                   �

��������������������������������������������������������������������������Ĵ

� D3Z/D7Z�  70    � O        �ICEN active :                                �

� or EMP �        �          � When nibble mode is selected (default),and  �

� *Note2 � (72)   �          � during the read command period. The pin out-�

�        �        �          � put data /bit3 or /bit7, high bit (nibble)  �

�        �        �          � first and then low bit. All other condition �

�        �        �          � the pin will output EMP status.EMP indicates�

�        �        �          � that the receive buffer ring contains data  �

�        �        �          � page is empty when high.                    �

�        �        �          �ICEN inactive :                              �

�        �        �          � The pin is in tri-state.                    �

��������������������������������������������������������������������������Ĵ

� DS0    �  66    � I        �Data strobes 0 :                             �

�        �        �          � Once DL3520  detects one of the DS0 falling �

�        � (68)   �          � edge transition, the data on CD0-CD7 port   �

�        �        �          � will be latchin when write or next data will�

�        �        �          � be strobed on CD0-CD7 or D0-D3/D4-D7 to be  �

�        �        �          � read when read data command is issued.      �

�        �        �          � Additional option information see appendix  �

�        �        �          � A.                                          �

��������������������������������������������������������������������������Ĵ

� DS1 or �  67    � I        �Data strobe 1 or READ/WRITE indicator :      �

� RD/WR  �        �          � When FAST mode it serve as RD/WR indicator, �

�        � (69)   �          � otherwise it function as DS1.               �

��������������������������������������������������������������������������Ĵ

� CS0    �  65    � I        �Command strobes :                            �

�        �        �          � Once DL3520  detects the falling edge tran- �

�        � (67)   �          � sition of CS0, the CD0-CD7 port will be     �

�        �        �          � latched as a command.                       �

��������������������������������������������������������������������������Ĵ

� ICEN   �  68    � I        �IC enable input :                            �

� *Note3 �        �          � When ICEN inactive,the DS0,DS1 and CS0 input�

�        � (70)   �          � transition detecters are disable,and printer�

�        �        �          � status port  will be tri-state except INT   �

�        �        �          � ( When /INTT is high,INT not in tri-state ) �

����������������������������������������������������������������������������

*Note1 : The Pin NO. has two value. One is for DL3520S small package

and the other with ( ) is for DL3520L large package.

*Note2 : When printer port FAST mode is selected, the pin

assignment of RDY and D3Z/D7Z/EMP is swaped.

*Note3 : ICEN signal can be setting to active high or active low

when power on reset. Additional information see appendix

A  "CEN" partion.

P.8                  D-LINK CONFIDENTIAL

4. EEPROM interface

��������������������������������������������������������������������������Ŀ

� Symbol � Pin No � Function �               Description                   �

��������������������������������������������������������������������������Ĵ

� ECS    �  10    � O        �EEPROM Chip select :                         �

�        �        �          � The signal when high select EEPROM to       �

�        � (12)   �          � operate data read/write ... functions.      �

��������������������������������������������������������������������������Ĵ

� ESK    �  9     � O        �Serial clock output to EEPROM :              �

�        �        �          � This port can generate clock signal to clock�

�        � (11)   �          � command or data on EDO pin into EEPROM.     �

��������������������������������������������������������������������������Ĵ

� EDO    �  8     � O        �Data output to EEPROM :                      �

�        �        �          � The pin serial out data to EEPROM DI pin    �

�        � (10)   �          � to issure a command or write data to EEPROM.�

��������������������������������������������������������������������������Ĵ

� EDI    �  7     � I        �Data input from EEPROM :                     �

�        �        �          � This pin is a input port. EEPROM output     �

�        � (9)    �          � data can be read via this port.             �

����������������������������������������������������������������������������

5. Power and system used signals.

��������������������������������������������������������������������������Ŀ

� Symbol � Pin No � Function �               Description                   �

��������������������������������������������������������������������������Ĵ

� Vdd    � 2,23   �  Power   � +5V DC is required. It is suggested that a  �

�        � 42,63  �          � decoupling capacitor be connected between   �

�        �(4,25   �          � Vdd and Vss pins.                           �

�        � 44,65) �          �                                             �

��������������������������������������������������������������������������Ĵ

� Vss    � 3,22   �  Power   � Vss = 0 Volt                                �

�        � 43,62  �          �                                             �

�        �(5,24   �          �                                             �

�        � 45,64) �          �                                             �

��������������������������������������������������������������������������Ĵ

� NIS0   �  19,   �  O       � Network interface select 0/1 :              �

� NIS1   �  18    �          � Those two user defined pins can be used as  �

�        � (21,   �          � network interface select control pin or     �

�        �  20)   �          � other usage.                                �

��������������������������������������������������������������������������Ĵ

�/RESET  �  16    �  I       � Chip reset :                                �

�        � (18)   �          � Active low to reset DL3520  .               �

��������������������������������������������������������������������������Ĵ

�/GATE   �  61    �  O       � CD bus external buffer enable control :     �

�        � (63)   �          � ICEN active  will active the signal low.    �

��������������������������������������������������������������������������Ĵ

� DIR    �  64    �  O       � CD bus direction :                          �

�        �        �          � The PIN indicates the direction of CD0-CD7  �

�        � (66)   �          � port. When high CD0-CD7 serve as a input,   �

�        �        �          � otherwise CD0-CD7 serve as a output port.   �

����������������������������������������������������������������������������

P.9                  D-LINK CONFIDENTIAL

5. Power and system used signals. ( continue )

��������������������������������������������������������������������������Ŀ

�/LNKS   �  17    �  I       � Link status input :                         �

�        �        �          �  When low indicates that twisted pair tran- �

�        � (19)   �          �  ceiver is in link condition.               �

��������������������������������������������������������������������������Ĵ

�/ALED   �  15    �  O       � LED indicator driver :                      �

�        �        �          �  steady on  -- ready to use. ( power on or  �

�        � (17)   �          �             link/cable connection properly) �

�        �        �          �  extinguish -- no power or bad cable        �

�        �        �          �                connection.                  �

�        �        �          �  blinking   -- data TX/RCV.                 �

��������������������������������������������������������������������������Ĵ

�/XLED   �  14    �  O       � POWER/TXM indicator driver :                �

�        �        �          �  steady on  -- ready to use. ( power on )   �

�        � (16)   �          �  extinguish -- no power .                   �

�        �        �          �  blinking   -- data TXM .                   �

��������������������������������������������������������������������������Ĵ

�/RLED   �  13    �  O       � LINK/RCV indicator driver :                 �

�        �        �          �  steady on -- link/cable connection properly�

�        � (15)   �          �  extinguish -- no power or above condition. �

�        �        �          �  blinking  -- data RCV .                    �

��������������������������������������������������������������������������Ĵ

�/CLED   �  12    �  O       � Collision indicator driver :                �

�        �        �          �  extinguish -- no power .                   �

�        � (14)   �          �  blinking   -- when collision occur.        �

��������������������������������������������������������������������������Ĵ

�/JLED   �  11    �  O       � Jabber indicator driver :                   �

�        �        �          �  extinguish -- no power .                   �

�        � (13)   �          �  blinking   -- when jabber occur.           �

��������������������������������������������������������������������������Ĵ

� PSS-   �  74,   �  I       � Power switch sensor -/+ :                   �

� PSS+   �  75    �          �  Both PSS- and PSS+ are internal pull high  �

�        �        �          �  . Whenever PSS- sense  high  level  or     �

�        � (76,   �          �  PSS+ sense low , the PSO will inactive im- �

�        �  77)   �          �  mediately. When PSS-/PSS+ sense low/high   �

�        �        �          �  simutaneously then PSO will delay seconds  �

�        �        �          �  to activate. See appendix A                �

��������������������������������������������������������������������������Ĵ

� PSO    �  60    �  O       � Power switch output :                       �

�        �        �          �  This signal output level is controlled by  �

�        � (62)   �          �  PSC signal.                                �

�        �        �          � Additional option information see appendix  �

�        �        �          � A.                                          �

��������������������������������������������������������������������������Ĵ

� PSC    �  59    �  I       � Power switch level control :                �

�        �        �          �  When input high, PSO output is low active. �

�        � (61)   �          �  Otherwise, it will be high active.         �

��������������������������������������������������������������������������Ĵ

�/TEST   �  6     �  I       � This pin is used for test only. When normal �

�        � (8)    �          � operation the pin must pull high.           �

����������������������������������������������������������������������������

P.10                 D-LINK CONFIDENTIAL

HOST INTERFACE :

Printer port and DL3520  printer port interface signals cross reference.

STATUS PORT

===========

11     10     12     13     15         <- PIN OF D-SUB CONNECTOR

Printer �������������������������������������������������������Ŀ

(STATUS)� BUSY � /ACK �  PE  � SLCT �/ERROR�  -   �   -  �   -  � ( 3BD h )

port    ���������������������������������������������������������

^

�

DL3520  �������������������������������������������������������Ŀ

NIBBLE  � EMP  �  INT � TXBZ1� TXBZ0� RDY  �  -   �   -  �   -  �

MODE    �������������������������������������������������������Ĵ

*Note4  �@D3/D7� D2/D6� D1/D5� D0/D4� RDY  �  -   �   -  �   -  �

���������������������������������������������������������

�������������������������������������������������������Ŀ

BYTE    � EMP  �  INT � TXBZ1� TXBZ0� RDY  �  -   �   -  �   -  �

MODE    ���������������������������������������������������������

�������������������������������������������������������Ŀ

FAST    � RDY  �  INT � TXBZ1� TXBZ0� EMP  �  -   �   -  �   -  �

MODE    ���������������������������������������������������������

*Note4 : In nibble mode, The port output are status except

in read data command period.

@ The data D3/D7 is inverted output in nibble mode.

DATA   PORT

===========

9      8      7      6      5      4      3      2

Printer �������������������������������������������������������Ŀ

(DATA)  � DATA7� DATA6� DATA5� DATA4� DATA3� DATA2� DATA1� DATA0� ( 3BC h )

port    ���������������������������������������������������������

�                     ^

� DIR=1 *Note5        � DIR=0

v                     �

DL3520  �������������������������������������������������������Ŀ

� CD7  � CD6  � CD5  � CD4  � CD3  � CD2  � CD1  � CD0  �

���������������������������������������������������������

*Note5 : When nibble mode selected DIR always equal 1 .

P.11                 D-LINK CONFIDENTIAL

COMMAND PORT

============

17     16      14      1

Printer ����������������������������������������������������������Ŀ

(CMD)   �  -   �  -   �  -   � IRQEN�/SLCTIN�/INIT �/AUTOFD�/STROBE� ( 3BE h )

port    ������������������������������������������������������������

�

DL3520                               v

BYTE or ����������������������������������������������������������Ŀ

NIBBLE  �  -   �  -   �  -   �  -   �  CS0  � ICEN �  DS0  �  DS1  �

MODE    ������������������������������������������������������������

����������������������������������������������������������Ŀ

FAST    �  -   �  -   �  -   �  -   �  CS0  � ICEN �  DS0  � RD/WR �

MODE    ������������������������������������������������������������

P.12                 D-LINK CONFIDENTIAL