Jul.1995

LANsmart Installation

for

D-Link DE-621 Duplex and DE-620 Ethernet Pocket LAN Adapter

===========================================================

Thank you for purchasing the D-Link Duplex for desktop, laptop

and notebook PCs to retain DE-620 and printer or key card connection.

Before you installing driver program, please refer to DE-620 or DE-621

manual to get more detail installation. A simple installation is as the

following figure.

����

/	/�

/    /  �

/    /	*�<------- connect to DE-620 only

����Ŀ  *  �

connect     |\ 	      �    �*	*�<------- connect to printer or

to pc	     |.|	  �����    �  *  /	   key card

printer <-- |.|���������ͳ DE-621 �*  /

port	     |.|	  �	   � /

|/ 	  ����������

This installation procedure applies to the following D-Link products:

DE-620,DE-620CT,DE-620TP,DE-620AUI,

Files	     Contents

-----------  -------------------------------------------------

DE-620.DRV   Driver program for the above-mentioned adapters

DE-620.EXE   Driver program for the above-mentioned adapters

1. RUN LANsmart version 2.xx & v3.xx

=====================================

You may use both the menu and the command mode to logon to the network.

The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:DE-620 [other parameters]

The driver program DE-620.DRV should first be copied to the directory

that contains LOGON.EXE program. /NET:DE-620 specifies to LANsmart your

network adapter card.