Jul. 1995

Ethernet Card Setup/Diagnostic program

for

DE-621 Duplex and DE-620 Ethernet Pocket LAN Adapter

====================================================

Thank you for purchasing the D-Link Duplex for desktop, laptop

and notebook PCs to retain DE-620 and printer or key card connection.

Before you installing driver program, please refer to DE-620 or DE-621

manual to get more detail installation. A simple installation is as the

following figure.

����

/	/�

/    /  �

/    /	*�<------- connect to DE-620 only

����Ŀ  *  �

connect     |\ 	      �    �*	*�<------- connect to printer or

to pc	     |.|	  �����    �  *  /	   key card

printer <-- |.|���������ͳ DE-621 �*  /

port	     |.|	  �	   � /

|/ 	  ����������

The SETUP620.EXE program (Setup/Diagnostic) lets you configure the DE-620

hardware settings, and test the state and cable connection of the card.

To run the program, insert the DE-620 driver program diskette in drive 'A' and

do the following:

> cd \UTILITY	     --change directory to UTILITY

> SETUP620	     --run the SETUP620.EXE program

or

> cd \UTILITY	     --change directory to UTILITY

> SETUP620 /F	     --run the SETUP620.EXE with parameter /F

if the data in your EEPROM has been corrupted

or you cannot configure your card through normal

Please run "SETUP620 /F".

Menu Options

============

1.  SETUP lets you configure the hardware settings of DE-620.

You have the option to view/change :

Node ID

Printer Port's Base Address

Printer Port's IRQ

Adapter Model

Connector

Printer Port's Mode

RAM Size

Printer Port Writes to Adapter

Printer Port Reads from Adapter

Runtime Configuration by

2.  DIAGNOSTIC lets you diagnose the state and cable connections of DE-620.

3.  DIAGNOSTIC lets you diagnose if DE-621 Duplex work with DE-620 is fine.