Jul. 1995

NetBIOS Emulator Installation

for

DE-621 Duplex and DE-620 Ethernet Pocket LAN Adapter

====================================================

Thank you for purchasing the D-Link Duplex for desktop, laptop

and notebook PCs to retain DE-620 and printer or key card connection.

Before you installing driver program, please refer to DE-620 or DE-621

manual to get more detail installation. A simple installation is as the

following figure.

����

/	/�

/    /  �

/    /	*�<------- connect to DE-620 only

����Ŀ  *  �

connect     |\ 	      �    �*	*�<------- connect to printer or

to pc	     |.|	  �����    �  *  /	   key card

printer <-- |.|���������ͳ DE-621 �*  /

port	     |.|	  �	   � /

|/ 	  ����������

The D-Link NetBIOS Emulator is a standardized network interface

program that allows this Adapter to run NetBIOS-based network

operating systems like the IBM PC LAN Program and others.

No software modification or additional installation is necessary.

All operations as well as installation procedures for the network

operating system will remain unaltered.

The following files will be used in this installation:

Files		  Contents

-----------	  -------------------------

DE-620.EXE	  DE-620 Driver program

NBIOS.EXE	  NetBIOS Emulator Program

To install the NetBIOS Emulator, do as follows:

1. Load in the driver program DE-620.EXE.

>DE-620

2. Load in the NetBIOS Emulator program NBIOS.EXE.  This program

is memory-resident and will stay in your computer's memory

until its power is turned off or until you re-boot.

>NBIOS [/Sm] [/Ck]

Typing the parameter after NBIOS is optional. The parameter is used

as follows:

/Sm   specifies the maximum number of sessions allowed on your

computer, where m can be a value from 1 to 254.  If this

parameter is not specified, the default number of sessions

will be set to 32.

/Ck   specifies the maximum number of NetBIOS command available for

your computer where k can be a value from 1 to 255.	If this

parameter is not specified, the default number of NetBIOS

commands will be set to 32.

Your computer now is ready to run a NetBIOS-based network operating system.