### DE-528CT

Ethernet Adapter

for PCI Bus

#### Installation Guide

### Introduction

Installation of a DE-528CT Ethernet Adapter requires hardware installation first, then software installation.

#### 1st Step – Unpack and Inspect

Open the shipping carton and carefully remove all items. In addition to this User's Guide, ascertain that you have:

· One DE-528CT Ethernet Adapter Card.

· DE-528CT Software Diskette.

. BNC T connector.

2nd Step -  Insert the adapter into an

expansion slot

1. Shut down the computer, unplug its power cord, and remove the chassis cover.

2. Insert the card into the connector of any open PCI Bus I/O expansion slot. Ascertain that the card is fully seated in the expansion slot connector.

3. Install the bracket screw which secures the card to the computer chassis.

4. Replace the computer s chassis cover.

#### 3rd Step – Connecter le câble

Before connecting a network cable to the DE-528CT adapter, ascertain that the network cable medium is Twisted-Pair or Thin Coaxial cable. Then refer to one of the two sections following, according to your network cable type.

TWISTED-PAIR CABLE

Ascertain that you have a straight twisted-pair cable, UTP cable (Cat. 3 ,4 ,5) or EIA/TIA-568 non-standard 100-ohm STP cable, not more than 100 meters (328 feet) in length for the network connection. The cable will have an RJ-45 plug on each end. The stations are connected in a star topology.

Plug one end into the RJ-45 receptacle of the DE-528CT adapter, and plug the other end into an available port of a 10Base-T hub. Push each RJ-45 plug into its receptacle until its retainer snaps into place.

THIN-COAXIAL CABLE

Ascertain that you have 0.2 inch diameter RG-58A/U 50-ohm coaxial cable, with BNC connectors at each end, not more than 185 meters (600 feet) in length for the network connection. The stations are connected in a bus topology. Follow the steps:

1. Connect a BNC T-connector onto the BNC receptacle of the DE-528CT card.

2. Connect the network cable onto either open end of the BNC T-connector.

3a. If the cable continues on to an additional station or stations, then connect the continuation cable onto the  emaining open end of the BNC T-connector.

3b. If the network cable does not continue to additional stations (your computer is at the end of the cable), then connect a 50-ohm terminator to the remaining open end of the BNC T-connector.

#### 4ème Etape – Installation du logiciel

On account of the great variety of network environments in which the DE-528CT may be installed and used, and the frequency of revisions in these network systems, the instructions for software installation are given as README files on the DE-528CT driver program diskette supplied in your DE-528CT package. Required software is provided on a diskette labeled the DE-528CT Installation. The diskette contains drivers for various NOSs (Network Operating System), and README files which provide instructions for installing the driver specific to your NOS.

Insert the DE-528CT Installation program diskette in an available diskette drive (here shown as drive A). To view related installation instructions, type in A: HELP.EXE and press Enter, then the Main Menu dialog box appears.

Choose Instruction to view the overview information, then for full installation details, choose the specific command which is appropriate to your network operating system.

To get the related driver from the DE-528CT Installation diskette, type in A: INSTALL. When D-Link DE-528CT Ethernet Adapter AUTOinstall Program dialog box appears, choose a command, such as Novell Netware Dos Client, then follow the instructions on your screen.