Model DGE-500SX

Full-Duplex

GIGABIT Ethernet Adapter

with SC-type Fiber Optic Interface, for PCI Bus

User’s Guide

Rev. 01 (August, 1999)

6DGE500SX.01

Printed in Taiwan

RECYCLABLE

Trademarks

Copyright 1999 D-Link Corporation.
Contents subject to change without prior notice.
D-Link is a registered trademark of D-Link Corporation/D-Link Systems, Inc.

All other trademarks belong to their respective proprietors.

Copyright Statement

No part of this publication may be reproduced in any form or by any means or used to make any derivative such as translation, transformation, or adaptation without permission from D-Link Corporation/D-Link Systems Inc., as stipulated by the United States Copyright Act of 1976.

Limited Warranty

Hardware:

D-Link warrants its hardware products to be free from defects in workmanship and materials, under normal use and service, for the following lengths of time from the date of purchase from D-Link or its Authorized Reseller:

| Product Type | Warranty Period |
| --- | --- |
| Network adapters | Lifetime |
| Unmanaged and managed hubs (10Mbps) | Lifetime * |
| Unmanaged and managed hubs (100Mbps) | One year |
| Repeaters, MAUs , transceivers, media converters | One year |
| Concentrators | One year |
| Internetworking products | One year |
| * Power supply and fans in these devices | One year |
| Other hardware products | One year |
| Spare parts and spare kits | 90 days |

If a product does not operate as warranted during the applicable warranty period, D-Link shall, at its option and expense, (1) repair the defective product or part, (2) deliver to Customer an equivalent product or part to replace the defective item.  All products that are replaced will become the property of D-Link. Replacement products may be new or reconditioned.  Any replaced or repaired product or part has a ninety (90) day warranty or the remainder of the initial warranty period, whichever is longer.

D-Link shall not be responsible for any software, firmware, information, or memory data of Customer contained in, stored on, or integrated with any products returned to D-Link pursuant to any warranty.

All products with lifetime warranty have a standard five-year warranty. To qualify for lifetime warranty, the enclosed Product Registration Card must be  completed and returned to D-Link within ninety (90) days of purchase.

Warranty service may be obtained by contacting a D-Link office within the applicable warranty period for a Return Material Authorization (RMA) number. If a Registration Card has not been previously sent, proof of purchase, such as a copy of the dated purchase invoice, must be provided.  Once an RMA number is issued, the defective product must be shipped back to D-Link prepaid, insured and wrapped in the original or similar shipping package to ensure that it will not be damaged during shipment.  When returning the defective product to D-Link for service, the RMA number must be marked on the outside of the shipping package.  Any product returned without an RMA number shall be rejected and sent back to the Customer, and D-Link reserves the right to have Customer bear the cost of sending back such products. A service charge may or may not be levied to Customer by D-Link.  To find out if a service charge is levied or not, and the charged amount, read the RMA that is returned to Customer, or ask the D-Link office when an RMA is requested.

Software:

D-Link warrants that the software programs licensed from it will perform in substantial conformance to the applicable published program specifications for a period of ninety (90) days from the date of purchase from D-Link or its Authorized Reseller.  D-Link warrants the magnetic media containing software against failure during the warranty period.  No updates are provided. D-Link's sole obligation hereunder shall be to replace any defective software products with products which substantially conform to D-Link's applicable published specifications.  Customer assumes responsibility for the selection of the appropriate applications program and associated reference materials. D-Link makes no warranty that its software products will work in combination with any hardware or applications software products provided by third party, that the operation of the software products will be uninterrupted or error free, or that all defects in the software product will be corrected. For any third party products listed in the D-Link software product documentation or specifications as being compatible, D-Link will make reasonable efforts to provide compatibility, except where the non-compatibility is caused by "bug" or defect in the third party's product.

Warranty service for software products may be obtained by contacting a D-Link office within the warranty period.  Where no Product Registration Card has been sent by Customer, proof of purchase, such as a copy of the dated purchased invoice, must be provided.

D-Link Offices to Contact for Warranty Service

To obtain an RMA number for warranty service, contact the D-Link office nearest you.  A list of contact addresses for D-Link’s international offices is found in the back of this User’s Guide.  Your Warranty Registration Card should also be sent to your regional D-Link office.

LIMITATION OF WARRANTIES

IF THE D-LINK PRODUCT DOES NOT OPERATE AS WARRANTED ABOVE, THE CUSTOMER'S SOLE REMEDY SHALL BE, AT D-LINK'S OPTION, REPAIR OR REPLACEMENT. THE FOREGOING WARRANTIES AND REMEDIES ARE EXCLUSIVE AND ARE IN LIEU OF ALL OTHER WARRANTIES, EXPRESSED OR IMPLIED, EITHER IN FACT OR BY OPERATION OF LAW, STATUTORY OR OTHERWISE, INCLUDING WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE. D-LINK NEITHER ASSUMES NOR AUTHORIZES ANY OTHER PERSON TO ASSUME FOR IT ANY OTHER LIABILITY IN CONNECTION WITH THE SALE, INSTALLATION MAINTENANCE OR USE OF D-LINK'S PRODUCTS

D-LINK SHALL NOT BE LIABLE UNDER THIS WARRANTY IF ITS TESTING AND EXAMINATION DISCLOSE THAT THE ALLEGED DEFECT IN THE PRODUCT DOES NOT EXIST OR WAS CAUSED BY THE CUSTOMER'S OR ANY THIRD PERSON'S MISUSE, NEGLECT, IMPROPER INSTALLATION OR TESTING, UNAUTHORIZED ATTEMPTS TO REPAIR, OR ANY OTHER CAUSE BEYOND THE RANGE OF THE INTENDED USE, OR BY ACCIDENT, FIRE, LIGHTNING OR OTHER HAZARD

LIMITATION OF LIABILITY

IN NO EVENT WILL D-LINK BE LIABLE FOR ANY DAMAGES, INCLUDING LOSS OF DATA, LOSS OF PROFITS, COST OF COVER OR OTHER INCIDENTAL, CONSEQUENTIAL OR INDIRECT DAMAGES ARISING OUT THE INSTALLATION, MAINTENANCE, USE, PERFORMANCE, FAILURE OR INTERRUPTION OF A D- LINK PRODUCT, HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY. THIS LIMITATION WILL APPLY EVEN IF D-LINK HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

IF YOU PURCHASED A D-LINK PRODUCT IN THE UNITED STATES, SOME STATES DO NOT ALLOW THE LIMITATION OR EXCLUSION OF LIABILITY FOR INCIDENTAL OR CONSEQUENTIAL DAMAGES, SO THE ABOVE LIMITATION MAY NOT APPLY TO YOU.

Table of Contents

Features	1

Unpack and Inspect	5

Installing the Adapter	5

Connecting the Network Cable	6

Software Installation	7

1

# Introduction

Thank you for choosing D-Link DGE-500SX, the value leader among Gigabit Ethernet adapters for PCIBus personal computers.

The DGE-500SX is designed to transmit and receive at the speed of one gigabit per second (1Gbps or1000Mbps) at full-duplex, while continuing support IEEE 802.3 CSMA/CD local area networks.

The DGE-500SX is used to connect a computer to a gigabit fiber switch or fiber module at full-duplex.

Hardware required for the DGE-500SX is Pentium II or above.

## Features

Designed for versatility and performance, the DGE-500SX Adapter provides the following features:

- Operates in a 32-bit or 64-bit PCI Local Bus Master slot of the computer.

- PCI Bus Master memory access, for high throughput .

- Supports PCI clock speed up to 66Mhz, independent of network clock.

- Complies with IEEE 802.3z 1000BASE-SX Gigabit Ethernet standards and IEEE 802.3 Ethernet standards.

- Support symmetric and asymmetric flow control defined by IEEE 802.3x and 802.3z.

- Plug-and-Play installation.

- Supports VLAN filtering.

- Multi-mode SC Optical Fiber connector for network cable connection.

- Cable lengths up to 220 meters using 1300nm 62.5/125 fiber optic cable.                                                            Cable lengths up to 500 meters using 1300nm 50/125 fiber optic cable.

- 1000Mbps Gigabit Ethernet data transfer rate.

- Full-duplex operation only.

- Built-in 96K FIFO buffers reduce overhead of memory transfers and optimize operating bandwidth, throughput, and network data integrity.

- Level-One LXT1001GMAC (Gigabit Ethernet MAC) controller which contains a built-in 64 bit, 33 MHz PCI interface.

- Four LED indicators: Full Duplex, Link and Transmit / Receive.

- Driver support for:

|  Novell Netware 4.11 |  Novell Netware 5.0 |
| --- | --- |
|  Microsoft Windows 95 NDIS4 |  Microsoft Windows 98 NDIS4 |
|  Linux |  Microsoft Windows NT 4.0 NDIS 4 |

2

# Installation

## Unpack and Inspect

CAUTION:	Under ordinary circumstances, a DGE-500SX card will not be affected by static charge as may be received through your body during handling of the unit.  In special circumstances where you may carry an extraordinarily high static charge, it is good practice to reduce the charge by touching a ground before handling the adapter card.

Open the shipping carton and carefully remove all items. In addition to this User's Guide, ascertain that you have:

- One DGE-500SX Ethernet Adapter Card.

- One 3.5" diskette:  DGE-500SX Software Diskette.

## Installing the Adapter

- Shut down the computer, unplug its power cord, and remove the chassis cover.

- Insert the contact edge of the DGE-500SX card into the connector of any available PCI Bus Master expansion slot.  Press the card firmly into the connector such that the card’s contacts are fully seated in the connector.

- Install the bracket screw which secures the card to the computer chassis.

- Replace the computer’s chassis cover.

- Reconnect the computer’s power cord, and switch computer power on.  If the BIOS section of your computer’s boot program is Plug-and-Play compliant, then at power-up the BIOS will automatically configure any newly installed DGE-500SX adapter.

NOTE: 	Due to a fault in some Plug-n-Play BIOS programs, it happens occasionally that a newly installed adapter is assigned an Interrupt Number which is already assigned to another device.  In such a case, the conflict of Interrupt Number will cause faults in the behavior of both devices.  Then it is necessary to run the CMOS Setup utility, and manually assign a non-conflicting Interrupt Number.

## Connecting the Network Cable

Multimode (62.5/125m or 50/125m) fiber cabling with an IEEE recommended SC connector is required for the adapter.  Use either one duplex SC connector, or two simplex SC connectors, and observe the correct relationship between the receive (Rx) and transmit (Tx) ports on the adapter and repeater.  If there is a need to monitor the adapter’s faceplate LEDs, make the rear of the PC or workstation accessible and leave sufficient clearance for cabling and service.

## Software Installation

Vast varieties of network architecture considered, the instruction for driver software installation are given as README files in the DGE-500SX Software Diskette.  Review the root directory README for overview information, and for full installation details, see the README and referenced instruction files in the subdirectory appropriate to your network operating system.

A

# Troubleshooting

If you experience any problems with the adapter, first: ascertain that the appropriate driver is loaded; that the proper grade of cable is employed for the network connection; and that the supporting hub is properly qualified for the application (such as 1000Base-SX).

The DGE-500SX Adapter features four LED indicators:

- FDX

- Steady green indicates the adapter is operating in    Full-Duplex mode.

- 1000M

The LED indicator lights green when a 1000 Mbps device is connected to the adapter.

- Link

- Steady yellow indicates good linkage between the DGE-  500SX and its supporting server or switch.

- TX/RX

- Flashing green indicates activity (transmitting or receiving) within the adapter.

B

# Specifications

- Jumperless Hardware

- Media interface: Optical Fiber Transceiver SC Connector

- LAN Chip Set:

- Interface controller: Level-One LXT1001GMAC

- Transceiver interface: HP-53D5E

- EMI Certifications:

- FCC Class B

- CE Certification, Class B

- Host interface:  PCI Bus (Bus Master)

- I/O base address assigned by Plug and Play system

- Interrupt Number Assigned by Plug and Play system

- Physical Dimensions: 175 mm x 94 mm cm

- Environment:

- Storage:	5 to 55C, (23 to 131 F)

- Operating: 0 to 50 C, (32 to 122 F)

- Humidity:  5% to 95% non-condensing

- Power Consumption: 7.8W

Registration Card

Print, type or use block letters.

Your name: Mr./Ms_______________________________________________________________________

Organization:_____________________________  Dept. _________________________________________

Your title at organization: __________________________________________________________________

Telephone:__________________________  Fax:_______________________________________________

Organization's full address: ________________________________________________________________

______________________________________________________________________________________

Country:______________  Date of purchase (Month/Day/Year):____________________________________

| Product Model | Product Serial No. | * Product installed in type of computer (e.g., Compaq 486) | * Product installed Computer serial No. |
| --- | --- | --- | --- |

(* Applies to adapters only)

Product was purchased from:

Reseller's name: ________________________________________________________________________

Telephone:_______________________  Fax:__________________________________________________

Reseller's full address: ___________________________________________________________________

______________________________________________________________________________________

1.	Where and how will the product primarily be used?

oHome oOffice oTravel oCompany Business oHome Business oPersonal

2. How many employees work at installation site?

o1 employee o2-9 o10-49 o50-99 o100-499 o500-999 o1000 or more

3. What network protocol(s) does your organization use ?

oXNS/IPX oTCP/IP oDECnet oOther_____________________________

4. What network operating system(s) does your organization use ?

oD-Link LANsmart oNovell NetWare oNetWare Lite oSCO Unix/Xenix

oPC NFS o3Com 3+Open oBanyan Vines oDECnet Pathwork

Windows NT Windows NTAS Windows '95 	Other______________

5. What network management program does your organization use ?

D-View HP OpenView/Windows HP OpenView/Unix SunNet Manager Novell NMS

NetView 6000 Other____________________

6. What network medium/media does your organization use ?

Fiber-optics Thick coax Ethernet Thin coax Ethernet

10BASE-T UTP/STP	100BASE-TX 100BASE-T4 100VGAnyLAN 	Other_________________

7. What applications are used on your network?

Desktop publishing Spreadsheet Word processing CAD/CAM

Database management Accounting Other_____________________

8. What category best describes your company?

Aerospace Engineering Education Finance Hospital Legal Insurance/Real Estate Manufacturing 	Retail/Chainstore/Wholesale Government Transportation/Utilities/Communication VAR 	Systemhouse/companyOther_____________

9. Would you recommend your D-Link product to a friend?

oYes  oNo (why?) ______________________    oI don’t know yet

10. Your comments on this product:

Offices

U.S.A.	D-LINK  SYSTEMS, INC.

53 Discovery Drive, Irvine, CA 92618  USA

TEL: 1-714-788-0805  FAX: 1-714-753-7033

CANADA	D-LINK  CANADA, INC.

2180 Dunwin Drive, Unit # 6, Mississauga Ontario, L5L 5M8, Canada

TEL: 1-905-828-0260  FAX: 1-905-828-5669

EUROPE (U.K.)	D-LINK (EUROPE) LTD.

D-Link House, 6 Garland Road, Stanmore, London HA7 1DP U.K.

TEL: 44-181-235-5555  FAX: 44-181-235-5500

GERMANY	D-LINK (DEUTSCHLAND) GMBH I.G.

Bachstrae 22, D-65830 Kriftel Germany

TEL: 49-6192-97110  FAX: 49-6192-971111

FRANCE	D-LINK FRANCE

Le FLORILEGE #2, Allee de la Fresnerie

78330 Fontenay Le Fleury France

TEL: 33-1-3023-8688  FAX: 33-1-3023-8689

SWEDEN	D-LINK SWEDEN

World Trade Center P. O. Box 70396, 107 24 Stockholm Sweden

TEL: 46-8-700-6211 FAX: 46-8-219-640

DENMARK	D-LINK DENMARK

Naverland 2 DK-2600 Glostrup Copenhagen, Denmark

TEL:45-43-969-040 FAX:45-43-424-347

SINGAPORE	D-LINK SINGAPORE

1 International Business Park,

#03-12 The Synergy,  Singapore 609917

EL : 65-774-6233  FAX: 65-774-6322

AUSTRALIA	D-LINK AUSTRALIA

Unit 16, 390 Eastern Valley Way Roseville, NSW 2069 Australia

TEL: 61-2-9417-7100  FAX: 61-2-9417-1077

NEW ZEALAND	D-LINK NEW ZEALAND

203A Hurstmere Road, Takapuna

Auckland, New Zealand

TEL: 64-9-488-0502  FAX: 64-9-488-0565

CHINA	D-LINK CHINA

15th Floor, Science & Technology Tower

No. 11, Baishiqiao Road, Haidian District, Beijing 100081  China

TEL: 86-10-68467106-9  FAX: 86-10-68467110

JAPAN	D-LINK JAPAN

10F, 8-8-15 Nishigotanda, Shinagawa-ku, Tokyo, 141 Japan

TEL: 81-3-5434-9678  FAX: 81-3-5434-9868

INDIA	D-LINK INDIA

Bombay Office : Plot No.5, Kurla-Bandra Complex Rd.

Off Cst Rd., Santacruz (E) Bombay - 400 098 India

TEL: 91-22-6172478  FAX: 91-22-6172476

TAIWAN	D-LINK TAIWAN

2F, No.233-2 Pao-Chiao Rd, Hsin-Tien, Taipei,Taiwan, R.O.C.

TEL: 886-2-916-1600  FAX: 886-2-914-6299

Check  http://www.dlink.com  for newest release of drivers.

Check http://www.dlink.com  for newest release of drivers.