TeleDrive® for Windows 95® Version

V2.43.014B-d ES336D Driver Release Notes

10/14/97

ESS Technology

Modem/Telephony

Company Confidential

About This Release

ESS Technology is providing updated Windows 95® drivers for the TeleDrive® family of modem chip products in this release. These drivers are complete updates to prior ESS releases for MS Windows 95®.

Release Information

Product:		TeleDrive® for Windows 95® Version V2.43.014B-d

Drivers:	ESS reference designs of ES336D: ES2820, ES2818/ES2819

OS:			Microsoft Windows 95®

Release Date: October 14, 1997

Release Description

Version V2.43.01B-d changes

The following changes have been made to this version of modem controller drivers for Microsoft Windows 95®:

- Set handset off-hook / on-hook detection time to 400ms rather than 1s before.

- Fixed the problem when both stations disconnect the audio channel at the same time.

Known Problems:

- Hyperterminal, sending a “. TXT” file to another ESS modem, which is running under PC+, DOS-box. Even though modems connect at 33600, transfer is very slow, around 2400bps. If we use Zmodem under the same conditions, transfer is very fast. We understand this is a limitation of Hyperterminal and similar Applications that give modems one character at a time, thus creating a large overhead. (Bug#1).

- ALB in Bell212. AT\N0B1W1+MS=69,0,1200,1200. Modem says error. for Bell 1200 in our DSP. Only Bell 103 is supported (+ms=64). DSP bug, we think. Also, when Cardinal set to B1+MS= 69, 0, 1200, 1200 calls ESS336 (&F) modems do not connect. This is same as MCT’s CCITT override test. (Bug#2).

- ESS modem has short train when connect to USR courier. DSP issue. (Bug#3).

- In DOS-box, a 62K .TXT Fox ASCII file transfer, takes about 3’30”, at a rate of 300 characters/sec, while modems are connected at 33600 with V.42bis. Using Z-Modem, same file transfer takes 24 sec (slow to fast PC) and 9” (Fast to slow). When paying DOOM a similar situation seems to exist - very slow. (Bug#4).

- Throughput vs. NMC. Test stopped at suite 004, test 39, and modems never reconnected. When test stopped modem B reported “Connect 9600”, no EC. Similar situation was reported by GVC. This has not been observed in 13E, but we keep it under watch. (Bug#5).

- Quite often ESS modems retrain during handshaking making the Connection cycle longer than other modems. Believed to be a DSP problem. (Bug#7).

- When starting game Doom, in Dos-box, our modem is identified as having a 8250 UART, rather than a 16550 UART. I understand this is by design. Game play is slow. (Bug#8).

- Force both modems to V.21 at 300 bps. At connection time, calling modem displays about ½ a line of garbled characters on its DTE. (Bug#9).

- DOS-Box. Use PC+ for DOS or QuickLink. Transfer a file from a fast to a slow PC, using ASCII (no protocol) and capture file. Data is lost. If you use Zmodem or similar protocol then all data is received fine. (Bug#10).

- Sometimes modem does not report RING after the first ring has happened. To reproduce this is useful to use +VDR=1 command. (Bug#11).

- On our .INF file we tried to disable S30 - Inactivity Timer and Inactivity Timer Scale. MCT still thinks we support this and considers that we pass both tests with a warning. Manual says S30 is reserved. We might want to support this because is one of the tests MCT performs. (Bug#12).

- During Connectivity test, on Tas System#2, when ESS modem calls USR Courrier V.Everything there is no connection, but when USR calls ESS modems connect ok. In Buffer mode, modems connect fine, no matter which modem initiates the call. (Bug#13).

- ESS cannot send Class 1 Fax when the Sending side uses WinfaxPro 8.0 and the Receiving side uses Chyenne’s Birware ver 10.0. Could be an Application issue. (Bug#14).

- Telephones we have trouble connecting to: 217-698-6087, 706-722-3361, 317-638-8129, 313-535-1466, 408-335-4595, 970-229-0246.(Bug#19).

- X0 S6 and S7. When blind dial enabled with X0, modem does see Busy and report NO CARRIER before S7 expires. Ex: AT X0 S6=10 S7=30 DT. Modem times out in about 18 seconds while other modems time out in about 40 seconds. (Bug#20).

- ES336 to ES336, class 2 fax, line speed 4800. Sometimes some of the pages are lost, like 4 pages out of 71 were not received. Application is Cheyenne ver 10.0. This problem under investigation. (Bug#21).

- When running FaxLab sometimes PC where ESS modem is installed locks up. This problem under investigation. (Bug#22)

- Transmit time delay longer than external modem (~ 1 sec.) for V.80.

- DCE controlled indication not actually reported by DSP.

- There is a popping sound at the end of each voicemail prompt.

- In speakerphone mode, phone cannot detect remote disconnection or dial tone.

- Inconsistent volume levels in DTMF, rings, greeting, and voice messages in speakerphone mode.  Its very low when Screening is on and when you listen to prompt or replay messages on the local side.

- Audio channel on / off too often may corrupt the duplex file transfer in some cases.

- Answer modem may stay in audio-on-ring state unless it hangs up.