TeleDrive® for Windows NT4.0®

V3.43NT004G-T, ES336T-NT Driver Release Notes

Jun 16, 1998

ESS Technology

Modem/Telephony

Company Confidential

About This Release

ESS Technology is providing updated Windows NT 4.0® drivers for the TeleDrive® family of modem chip products.

Release Information

Product:		TeleDrive® for Windows NT4.0®, V4.43NT004G-T.

Drivers:	ESS reference designs of ES336T-NT: ES2820 with ES2818/ES2819

OS:			Microsoft Windows NT4.0®

Release Date: June 16, 1998

Release Description

Installation:

See README.TXT in the installation disk.

Windows NT4.0 does not have full support for PNP. When you install the modem, please, make sure driver resources used by ESS modem (Interrupt, I/O, etc.) are not conflicting with other devices. The default interrupt is 9.

Known Problems:

1.  Closed Procomm Plus for Windows and tried to bring up Hyperterminal.

Could not talk to modem. This is not a driver problem, but a Procomm

problem. Several Win95 applications may have bugs working under NT.

- When upgrading the driver, sometimes System may report “Could not stop the EDSP Port Driver service on (PC name). Error 2186: the service is not responding to the control function”. In this case you may have to retry or restart your PC.

- No Bell212 support.

- Quite often modems retrain during handshaking. Believed to be a DSP problem.

- No DOS-box support.

- Force both modems to V.21 at 300 bps. At connection time, calling modem displays about 1 to 2 lines of garbled characters on its DTE.

- Performance tests on TAS show results comparable to WIN95 drivers. There is a small chance (less than 1%) that connection on 17C.1 or 17C.2 will fail.

- Xon/Xoff is not properly implemented. Local modem is passing Xon/Xoff to the Remote, and Local side is not acting on Xon/Xoff (Keeps dumping data to Local Terminal after receiving Xon from DTE). In spite of this, we did not see any problems with ASCII file transfer when using software flow control.

- When connected in buffered mode, sometimes, needs up to 45 seconds to disconnect, after remote side drops line. Usually disconnects in less than 20 seconds. Same situation for Win95.

11.Volume L0-L3 does not change. This is by design.

12.Description of S37 in Manual is not accurate. Win95 and NT drivers

match each other and have 16 for 16800 etc., up to 22 for 33600. There

are no 23 or 24 choices.

13.On very rare occasions, modem might do ALB at 300, rather than V.34.

14.Remote configured to &F W1 S0=1. Calling modem &F +MS = 1, 0,

1200,1200. Sometimes Remote reports Carrier 300 and connection fails.

15.With PC+ ver 2.11, in V22bis and V22, full duplex, looks like some

loss of data occurs, but when we use HyperAccess there is no problem.

We think PC+2.11 problem.

16.Remote modem might report Ring, but handshake might not finish. In

this case Remote will stay off hook until S7 times out.