TeleDrive® for Windows 95®

V4.43.005C-I ES56-I Driver Release Notes

Sep 8, 1998

ESS Technology

Modem/Telephony

Company Confidential

About This Release

ESS Technology is providing updated Windows 95® drivers for the TeleDrive® family of modem chip products in this release. These drivers are complete updates to prior ESS releases for MS Windows 95®. X2 is not supported in this code version.

Release Information

Product:		TeleDrive® for Windows 95® Version V4.43.005C-I

Drivers:	ESS reference designs of ES56-I: ES2890 with ES2818

OS:			Microsoft Windows 95®

Release Date: Sep 8, 1998

Release Description

Version V4.43.005C-I

Known Problems:

- Hyperterminal, sending a TXT file to another ESS modem, which is running under PC+, DOS-box. Even though modems connect at 33600, transfer is very slow, around 2400bps. If we use Zmodem under the same conditions, transfer is very fast. This is a limitation of Win95 Hyperterminal and similar Applications that give modems one character at a time, thus creating a large overhead.

- No Bell212 support.

- Quite often modems retrain during handshaking.

- Games “Ultimate DOOM for Windows” and “DOOM for DOS” play very slow.

- Force both modems to V.21 at 300 bps. At connection time, calling modem displays about 1 to 2 lines of garbled characters on its DTE.

- DOS-Box. Use PC+ for DOS or QL2. Transfer a file using ASCII (no protocol) and capture it. Sometimes, transfer does not complete. If you use Zmodem or similar protocol transfer completes just fine.

- Set to +MS=17,0,33333,33333. Expected “Carrier 33333”, but got “Carrier 36000” instead. Similar situations for other choices.

- Performance, ESS to ESS. Throughput vs. NMC is ok. Connectivity, on 17C1 and 17C2, might show anywhere from 1 to 3 failures.

- XON/XOFF is not properly implemented in Windows and DOS-box drivers. Local modem Is passing XON/XOFF to the Remote and Local side is not acting on XON/XOFF (Keeps dumping data to Local Terminal after receiving XOFF from DTE). In spite of this, we did not see any problems with ASCII file transfer when using software flow control.

- When transfering a long ASCII file using PCP for DOS, with RTS/CTS

flow control on, PC+ might turn RTS off and never turn it back on. In

such case file transfer will fail and you may have to restart the

application.

11.When using WinFax 8.0 to send to USR, Supra or Motorola, which are

under BitWare 10.0 control, modems always fail to negotiate phase B.

Believed to be an Application problem.

12.Occasionally, modem might report Connect 300 when a higher

connection rate is expected.

13.When connected in buffered mode, sometimes needs up to 45 seconds

to disconnect, after remote side drops line. Usually disconnects in less

than 20 seconds.

14.Description of S37 is not accurate. Win95 and NT drivers match each

other and have selections 16 for 16800 etc., up to 22 for 33600. Ignore

choices 23 and 24.

15.With PC+ ver 2.11, in V22bis and V22, full duplex, looks like some

loss of data occurs, but when we use HyperAccess there is no problem.

We think PC+2.11 problem.