TeleDrive® for Windows 95® Version

V3.43.004i-T-X ES56T-X Driver Release Notes

April 30, 1998

ESS Technology

Modem/Telephony

Company Confidential

About This Release

ESS Technology is providing updated Windows 95® drivers for the TeleDrive® family of modem chip products in this release. These drivers are complete updates to prior ESS releases for MS Windows 95®.

Release Information

Product:		TeleDrive® for Windows 95® Version V3.43.003i-T-X

Drivers:	ESS reference designs of ES56T-X: ES2820 with ES2818/ES2819

OS:			Microsoft Windows 95®

Release Date: April 30, 1998

Release Description

Version V3.43.003i changes

The following changes have been made to this version of modem controller drivers for Microsoft Windows 95®:

- Increased dial buffer size.

- V.42 upgrade.

- PTT first dial delay.

- Voice V.80 support – detect tones while playing message.

- V32. Fixed “No Dialtone” reporting.

- V34. Handles response to short train when requested in first 5 seconds.

- No X2-V34 fall back once connected to X2.

Known Problems:

- Hyperterminal, sending a TXT file to another ESS modem, which is running under PC+, DOS-box. Even though modems connect at 33600, transfer is very slow, around 2400bps. If we use Zmodem under the same conditions, transfer is very fast. This is a limitation of Win95 Hyperterminal and similar Applications that give modems one character at a time, thus creating a large overhead.

- No Bell212 support.

- Quite often modems retrain during handshaking.

- Games “Ultimate DOOM for Windows” and “DOOM for DOS” play very slow.

- Force both modems to V.21 at 300 bps. At connection time, calling modem displays about 1 to 2 lines of garbled characters on its DTE.

- DOS-Box. Use PC+ for DOS or QL2. Transfer a file using ASCII (no protocol) and capture it. Sometimes, transfer does not complete. If you use Zmodem or similar protocol all data is received fine.

- Set to +MS=17,0,33333,33333. Expected “Carrier 33333”, but got “Carrier 36000” instead. Similar situations for other choices.

- Performance, ESS to ESS. Throughput vs. NMC is ok. Connectivity, on 17C1 and 17C2, might show anywhere from 1 to 5 failures.

- XON/XOFF is not properly implemented in Windows and DOS-box drivers. Local modem Is passing XON/XOFF to the Remote and Local side is not acting on XON/XOFF (Keeps dumping data to Local Terminal after receiving XOFF from DTE). In spite of this, we did not see any problems with ASCII file transfer when using software flow control.

- When transfering a long ASCII file, using PCP for DOS, with RTS/CTS flow control on, PC+ might turn off RTS and never turn it back on. In such a case, file transfer will fail and you may have to restart the Aplication.

11.Try to use SoundRecorder to play original WIN95 WAV files. Says

“Your hardware cannot play files like the current file”. After we open ESSPhone then can play WIN95 WAV files and also WAV files recorded using ESSPhone. If we close ESSPhone we no longer can play WIN95 ESS files.

12.If, under ControlPanel\Multimedia, both Playback and Recording are

set to “Voice Modem Wave #00 Handset”, we can leave and/or retrieve messages from a remote PC and play messages left on the Local PC or play Pre-Recorded Greetings to a telephone set. If Multimedia is set to “Voice Modem Wave #00 Line”, we can do all of the above except play Pre-Recorded Greetings to Local phone. If user changes Multimedia settings from Handset to Line, the PC has to be restarted before the changes take affect. In short, when using ES56T-X, user should select “Voice Modem Wave #00 Handset” to get maximum advantage of modem capabilities.

13.MSPhone and MSPhone Dialer can use ES56T-X to dial out but, if

Local user picks up TelSet after Remote has picked up, parties cannot talk to  each other. To establish a good phone path, Local user has to pickup PhoneSet and then click “Hangup” on MSPhone. This is in part because of hardware design, but mostly because of of the way MSPhone behaves.

Under WinNT 4.0, Microsoft has improved MSPhone design and there is no problem.

14.When using WinFax 8.0 to send to USR, Supra or Motorola, which are

under BitWare 10.0 control, modems always fail to negotiate phase B.

Believed to be an Application problem.