Compaq Fibre Channel Tape Controller I

Firmware Upgrade – c9909p

NOTE:  This code revision requires SmartStart 4.50 and Compaq Insight Manager 4.50.

Requirements:

1.  No more than two (2) DLT 35/70 tape drives per SCSI Bus.

2.  Tape Library SCSI IDs must be set as follows:

SCSI ID 1 = Robot (if applicable)

SCSI ID 2 = DLT Drive 1

SCSI ID 3 = DLT Drive 2

3.  Terminate all SCSI Buses, used or unused.

4.  If a Compaq Fibre Channel Tape Controller I and Compaq Fibre Channel Tape Controller II  reside on the same Fibre Channel Arbitrated Loop (FCAL), both must be running the same revision of firmware.

Upgrade Instructions:

- Firmware can be upgraded via the serial port.  Connect a workstation or laptop to the serial port on the back of the FCTC-I.   Open Hyperterminal (only on Windows95/98/NT) and create a new connection.  Communication parameters should be set as follows: 9600 baud, 8 data bits, 1 stop bit, no parity, no flow control.

- Press <Return> several times to initiate the Hyperterminal connection. From the main menu, type 'maint' to enter Maintenance Mode.  Select the option to 'Upgrade to a New Revision of Firmware'.  The FCTC-I will respond with:

This will replace the current revision of the firmware.  A reboot will also be performed as part of this process.  Are you sure (Y/N)?

Press Y to proceed with the upgrade.

The FCTC-I will respond with: Please send the firmware image.

- From the Hyperterminal Transfer pull-down menu, select Send File.  Select the Browse button and select the firmware image file.  Select Xmodem as the protocol.  Click Send to begin the transfer.

- The file transfer will take 10-12 minutes to complete.  At the completion of the transfer, the FCTC-I will respond with:

D O W N L O A D    C O M P L E T E

The FCTC-I will perform a warm reboot to complete the process and load the new firmware image.