The PMPFIX10.EXE program corrects a problem that I have found when I

upgraded my motherboard.  Both Poor Man's Packet (PMP11) and Packet

Monitor V1.2 (PKTMON12) expect the PC system timer to be in mode 3

before they are run and both do not initialize the timer into that mode.

Some motherboards or utilities leave the timer in mode 2 which causes

a timing problem for programs like PMP that read the timer on the fly.

In mode 2, the timer seems to run much slower, about half speed of mode

3, although the time of day interrupt is the same for both modes.

I have included both the C++ source file and the executable file.  They

are freely distributable.

When the PMPFIX10 program is executed, it will attempt to read the status

of all 3 timers, and set timer0 into mode 3 if necessary, and inform

the user of its progress.  After execution, PMP or PKTMON12 may be

run.

The program should work on an PC AT or higher class machine.

Any comments/questions may be directed to myself:

Jim Olson      Internet Email: xolson@svpal.org