EFFECT10.EXE DESCRIPTION

------------------------

Jos Groot, November 7, 1993

This file describes the first release of the program EFFECT10,  which  can

be used to add real time sound effects (for example,  an  echo)  to  sound

input to a Sound Galaxy NX Pro 16 soundcard or compatible.

1. INTRODUCTION

---------------

The Sound Galaxy NX Pro 16 (SGPRO16) is a 16-bit  stereo  sound  card.  It

contains a chip, the AD1848, which can sample and playback up  to  48  kHz

sample rate. EFFECT10 accesses this chip  directly  to  create  the  sound

effects. It uses the chip in its mono mode.

The Microsoft Windows Sound System uses this  chip  also.  I  expect  that

EFFECT10 functions with this card. However, I didn't  test  it.  The  same

holds for all other cards which claim (hardware)  compatibility  with  the

Microsoft system, or use an AD1848. EFFECT10 uses an  initialization  file

which includes the IO addresses used, which probably should be changed  to

use other cards (see below).

Section 5 of this file contains some additional information on the  AD1848

for those interested.

The program was written in Turbo Pascal 6.0 on a  33  MHz  80386  machine,

with an ordinary IDE (AT) IO  bus  running  at  11  MHz.  All  performance

figures given were measured on this machine. I expect the program to  work

on all 80286 and higher IDE bus computers with a VGA card.

2. GETTING STARTED

------------------

Start EFFECT10. One of the first things the  program  does  is  trying  to

detect the AD1848 of the SGPRO16. If it does, no problem. If  it  doesn't,

the program possibly uses the wrong IO addresses. These are read from  the

configuration file EFFECT10.CFG. Page A-7 of the Sound  Card  User  Manual

(version 1.1) of your SGPRO16 documentation contains some  information  on

the IO addresses used. It uses 8 contiguous bytes beginning at one of  the

following base addresses WAVBase: $530 (default), $E80, $F40 or $604  (the

$ is the Turbo Pascal method for writing hexadecimal  numbers).  The  base

address used is selected with the jumpers J6 and J7  (page  I2-10  of  the

User Manual). EFFECT10.CFG contains 4 addresses, because the  AD1848  uses

only 4 of the 8 IO addresses. These are:

Address   |  Description      | Default (WAVBase= $530)

-----------------------------------------------------------

WAVBase + 4H | Codec Address     |        $534

WAVBase + 5H | Codec Data        |        $535

WAVBase + 6H | Codec Status      |        $536

WAVBase + 7H | Codec Direct data |        $537

The AD1848 is referred to as the Codec (coder/decoder) in this table.  The

configuration file of this archive assumes  use  of  the  default  WAVBase

address, $530. If your card is configured  differently,  change  the  four

entries in the configuration file accordingly.

3. PROGRAM OPTIONS

------------------

The main menu presents some commands which are selected  by  pressing  the

first letter of the command. The next sections describe each  of  them.  I

suggest that you also work through all the program options in this order.

3.1 Report configuration

------------------------

Shows the figures read from the configuration file EFFECT10.CFG, and  some

other more or less useful info. Use it to  check  that  the  program  read

EFFECT10.CFG correctly.

3.2 Sample rate

---------------

The AD1848 of the SGPRO16 can operate at 14 different software  selectable

sample rates (frequencies), ranging from 5.5 to 48 kHz. Select one here.

Because the exact frequency depends on the crystals used, the frequency is

measured by the program. This value is used for all computations.  As  far

as I know, the  SGPRO16  uses  the  crystals  recommended  in  the  AD1848

datasheets, so the 14 frequencies  shown  by  EFFECT10  are  correct.  All

algorithms of the program will operate at the measured frequency.

The default sample rate can be changed in the configuration file.

3.3 Plot

--------

This option plots the signal, or  its  frequency  spectrum.  Connect  your

musical instrument (for example, an electric  guitar)  or  the  microphone

supplied with your SGPRO16 to the microphone input of the card. Select the

Plot option.

The top of the screen contains the current sample rate, the x- and  y-axis

range and units, and some statistical information. The y-axis range can be

changed in the configuration file ("Voltage for full ADC input").  If  you

have a tone generator and an oscilloscope  you  can  measure  the  correct

value. Substituting this value in the configuration file turns your  sound

card into a calibrated slow digital scope. The statistical information  is

the minimum, maximum, average and RMS (root-mean-square) value of the plot

below. The RMS value can be used to estimate the amount of noise  when  no

(or a constant) signal is input.

At the center is a plot of 640 samples. If you make your instrument sound,

or whistle into the microphone the plot  should  change.  If  it  doesn't,

check the connections. If the signal is  too  large  (small),  you  should

decrease (increase) the input gain entry in the configuration file.

Below the plot is a sub-menu with several options:

Statistics: turns the statistical information at the top on or off.

Decrease/Increase delay: the plot is refreshed every X seconds. The  delay

X can be changed with this option.

Frequency spectrum: if you  select  this  option  a  512  point  frequency

spectrum is plotted. Now you can for example see how the place at  which

you hit a guitar string affects the spectral content of the sound  (this

affects the timbre, not  the  pitch).  I  myself  discovered  that  some

annoying high noise from my computer had a frequency  of  9.3  kHz.  The

y-direction  scale  factor  for  this  plot  can  be  altered   in   the

configuration  file,  if  you  wish.  The  statistical  information  now

indicates the 4 frequencies with the highest intensity.  Note  that  the

x-direction scale changes with the sample  rate:  the  highest  spectrum

frequency is half the sample rate.

Non-continuous mode: selecting this option lets one determine when to take

and plot the samples or a spectrum  ("Next  frame"  option).  The  other

options of the changes sub-menu allow  you  to  measure  the  plot  more

accurately and to save it to the file EFFECT10.PLT.

Main menu: returns one to the main menu.

3.4 Distortion

--------------

Now its time to get some sound output. Connect an amplifier or speaker  to

the speaker/headphone connector of the SGPRO16 card. Select the Distortion

option and set the level to 0. The only thing the program does is  reading

ADC sample values and feeding them to the DAC, without  altering  them  at

all. You should now hear the sound  you  input  to  the  card.  If  it  is

distorted there are two possible causes: if the input level is  too  high,

adjust your instrument volume, or decrease the input  gain  entry  in  the

configuration file. If the output level is too high, adjust  it  with  the

volume control on your card, or decrease the DAC attenuation entry in  the

configuration file.

My machine is not fast enough to run this algorithm at 48 kHz. At  48  kHz

clicks are audible, because the program is sometimes too late  to  read  a

sample from the ADC. The highest  sample  rate  that  doesn't  cause  this

problem is 32 kHz. If you have a very slow machine, the number of  samples

it misses is so large that it causes  the  sound  to  be  distorted  quite

heavily.

Setting the distortion level to a  level  higher  than  0  will  create  a

distorted sound. I use quite a naive algorithm. For a  certain  distortion

level (Level) the value input to the DAC (In) is related to the ADC output

(Out) by:

(1 - Level/10)

( abs(In) )

Out = 32767 * sign(In) * ( ------- )

(  32767  )

Both In and Out are signed 16-bit integers. The use of this function  also

compresses the signal: small volume signals are amplified more than  large

ones.

3.5 Echo

--------

The maximum delay time is determined by the circular buffer length  (30000

samples) and the sample rate. It is:

maximum delay (in seconds)= 30000 / sample rate (in Hz)

The echo and direct volume can be adjusted separately.

3.6 Chorus

----------

The chorus effect results from adding a delayed  signal  to  the  original

one. The delay of the delayed signal is  automatically  varied  between  a

minimum and maximum value (typically around 50 ms), with  a  certain  rate

(typically less than 1 Hz). The volume of  the  delayed  signal  added  is

determined by the depth parameter.

Flanging and phasing are as far as I know essentially the same effect, but

with a delay less than 5 ms.

Here are some possibilities to try:

parameter          | chorus | flanger | weird

---------------------------------------------

minimum delay [ms] | 40     | 0       | 0

maximum delay [ms] | 60     | 5       | 100

rate          [Hz] | 0.25   | 0.5     | 4

depth              | 5      | 10      | 4

direct volume      | 10     | 10      | 10

3.7 Quit

--------

Quits the program.

4. REMARKS

----------

16-bit sound cards are often advertised  as  offering  CD  quality  sound,

because compact disc players use 16 bit samples at 44.1 kHz. This is often

not true. Real 16-bit sound quality  results  only  if  all  16  bits  are

measured and handled correctly. This means that  the  noise  and  harmonic

distortion should be very small. For the AD1848 the THD+N (Total  Harmonic

Distortion + Noise) figure is typically -70 dB for the DAC, and -76 dB for

the ADC (referenced to full scale). This means that about 12 bits  of  the

16 bits are reliable (1 bit accounts  for  20*log10(2)=  6�dB).  For  real

16-bit sound quality the THD+N figure is -96 dB. All  sound  effects  that

add samples (e.g., echoing) cause a  further  degradation  of  the  THD+N,

because the resulting sound includes the noise from all added samples.

For speed reasons the keyboard  isn't  checked  often  while  running  the

effects algorithms. It may take a while before the computer responds.

My  card  uses  the  J-grade  AD1848:  the  AD1848JP.  Production  of  its

successor, the AD1848KP, was scheduled for the second quarter of 1993. The

AD1848KP is a performance enhanced version of the AD1848JP. There are some

minor differences between programming the J- and K-grade. I did  not  test

my program with an AD1848KP based card, so I'm not absolutely sure that my

program handles the K-grade correctly.

Any comments on the program are welcome. I  am  especially  interested  in

making the program work  with  AD1848  based  soundcards  other  than  the

SGPRO16. Your comments will contribute to an improved future  release.  If

you have very good, but time consuming  ideas  for  a  better  (mouse  and

window based) user interface, or for other sound effects: I am willing  to

start a joint project on creating a future release.

Improvements I am considering are:

- add pitch shifting.

- avoid clicks caused by jumping from  the  last  to  the  first  circular

buffer element during the various buffer based effects.

- add a possibility to use a file with preset effects  (short  echo,  long

echo, chorus, flanger etc.).

5. AD1848 DETAILS

-----------------

This is the Product Overview part of the AD1848 data sheets:

The Parallel-Port AD1848 SoundPort Stereo Codec integrates the  key  audio

data conversion and control functions into a  single  integrated  circuit.

The AD1848 is intended to provide a complete, single-chip  audio  solution

for business audio and multimedia applications requiring operation from  a

single +5 V or mixed +5 V analog and +3.3 V digital supplies. It  provides

a direct, byte-wide interface to both ISA ("AT") and EISA  computer  buses

for simplified implementation on a computer mother-board or  add-in  card.

The AD1848 generates enable and direction controls for IC buffers such  as

74_245.

The  AD1848  SoundPort  Stereo  Codec   supports   a   DMA   request/grant

architecture for transferring data with the host computer bus. One or  two

DMA channels can be supported. Programmed I/O (PIO) mode is also supported

for control register access and for applications lacking DMA control.  Two

input control lines  support  mixed  direct  and  indirect  addressing  of

twenty-one internal control registers over this asynchronous interface.

External circuit requirements are limited to a minimal number of low  cost

support components.  Anti-imaging  DAC  output  filters  are  incorporated

on-chip. Dynamic range exceeds 80 dB over the 20 kHz  audio  band.  Sample

rates from 5.5 kHz to 48 kHz are supported from external crystals.

The  Codec  included  a  stereo  pair  of  Sigma-Delta   analog-to-digital

converters and a stereo pair of Sigma-Delta digital-to-analog  converters.

Inputs to the ADC can  be  selected  from  four  stereo  pairs  of  analog

signals: line, microphone ("mic"), auxiliary line #1, and  post-mixed  DAC

output. The microphone inputs can pass through optional 20 dB gain blocks.

A software-controlled programmable gain stage allows independent gain  for

each channel going into the ADC. The ADCs' output can be  digitally  mixed

with the DACs' input.

The pair of 16-bit outputs from the ADC  is  available  over  a  byte-wide

bidirectional interface that also supports 16-bit  digital  input  to  the

DACs and control information. The AD1848 can accept  and  generate  16-bit

twos complement PCM linear digital  data,  8-bit  unsigned  magnitude  PCM

linear data, and 8-bit u-law or A-law companded digital data.

The Sigma-Delta DACs are preceded by a digital  interpolation  filter.  An

attenuator provides independent user volume control over each DAC channel.

Nyquist images and shaped quantization noise are removed  from  the  DACs'

analog stereo output by  on-chip  switched-capacitor  and  continuous-time

filters. Two stereo pairs of auxiliary line-level inputs can also be mixed

in the analog domain with the DAC output.

6. DISCLAIMER

-------------

I did my utmost best to exclude  all  errors  from  the  contents  of  the

EFFECT10.ZIP archive. However, because I am just like you only human,  the

described program might not work or, even  worse,  cause  damage  to  your

equipment.  Nevertheless,  I  assume  no  liability  for  this.  The  only

guarantee I can give you is that the program functions up  to  expectation

with my configuration.

7. AUTHOR

---------

Author  : Jos Groot

Address : TNO-FEL

P.O. Box 96864

2509 JG  The Hague

The Netherlands

Phone   : 31 70 3264221

FAX     : 31 70 3280961

Internet: jos.groot@fel.tno.nl