PC 99 System Design Guide

A Technical Reference
for Designing PCs and
Peripherals for the Microsoft® Windows®
Family of Operating
Systems

Intel Corporation and Microsoft Corporation

The information contained in this document represents the current view of Intel Corporation and Microsoft Corporation on the issues discussed as of the date of publication. Because Intel and Microsoft must respond to changing market conditions, it should not be interpreted to be a commitment on the part of Intel and Microsoft, and Intel and Microsoft cannot guarantee the accuracy of any information presented. This document is for informational purposes only. INTEL AND MICROSOFT MAKE NO WARRANTIES, EXPRESS OR IMPLIED, IN THIS DOCUMENT.

Intel Corporation and Microsoft Corporation may have patents or pending patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. The furnishing of this document does not give you any license to these patents, trademarks, copyrights, or other intellectual property rights.

Intel and Microsoft do not make any representation or warranty regarding specifications in this document or any product or item developed based on these specifications. Intel and Microsoft disclaim all express and implied warranties, including but not limited to the implied warranties of merchantability, fitness for a particular purpose, and freedom from infringement. Without limiting the generality of the foregoing, Intel and Microsoft do not make any warranty of any kind that any item developed based on these specifications, or any portion of a specification, will not infringe any copyright, patent, trade secret, or other intellectual property right of any person or entity in any country. It is your responsibility to seek licenses for such intellectual property rights where appropriate. Intel and Microsoft shall not be liable for any damages arising out of or in connection with the use of these specifications, including liability for lost profit, business interruption, or any other damages whatsoever.

Active Desktop, BackOffice, Direct3D, DirectDraw, DirectInput, DirectMusic, DirectPlay, DirectShow, DirectSound, DirectX, Microsoft, Microsoft Press, MS-DOS, NetShow, WebTV, Win32, Windows, Windows NT, and the Windows logo are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries.

Indeo, Intel, and Pentium are registered trademarks of Intel Corporation.

Other product and company names herein may be the trademarks of their respective owners.

© 1998 Intel Corporation and Microsoft Corporation. All rights reserved.

BEGIN   BREAK

END   BREAK

Welcome			xi

Contents

Part 1  System Design Issues

Chapter 1	PC 99 Design Issues	3

Chapter 2	PC 99 Design Initiatives	15

Part 2  PC 99 Systems

Chapter 3	PC 99 Basic Requirements	39

PC 99 General System Requirements	40

PC 99 Physical Design Requirements	51

PC 99 General Device Requirements	53

PC 99 Buses and Devices	64

Manageability Component Instrumentation Requirements	75

PC 99 System References	77

Checklist for PC 99 Basic Requirements	79

Chapter 4	Workstation PC 99	83

Workstation Design Requirements	84

Workstation PC 99 References	87

Checklist for Workstation PC 99	88

Chapter 5	Entertainment PC 99	89

Entertainment PC System Requirements	90

Entertainment PC Audio Requirements	92

Entertainment PC Graphics Components	93

Entertainment PC Video and Broadcast Components	94

Entertainment PC 99 References	95

Checklist for Entertainment PC 99	96

Chapter 6	Mobile PC 99	97

Mobile PC System Design Requirements	98

Mobile PC Graphics Requirements	106

Docking Station Requirements	110

Mini-notebook Guidelines	120

Mobile PC 99 References	121

Checklist for Mobile PC 99	122

Part 3  PC 99 Bus Design Guidelines

Chapter 7	USB	127

USB Basic Requirements	128

USB Host Controller Requirements	130

USB Hub Requirements	130

USB Power Management	131

Design Features for USB Peripherals	131

USB References	132

Checklist for USB	133

Chapter 8	IEEE 1394	135

IEEE 1394 Basic Requirements	136

Requirements for IEEE 1394 Devices	138

Plug and Play for IEEE 1394	139

Power Management for IEEE 1394 Devices	149

IEEE 1394 References	150

Checklist for IEEE 1394	151

Chapter 9	PCI	155

PCI Basic Requirements	156

PCI Controller Requirements	157

Plug and Play for PCI Controllers and Peripherals	158

Power Management for PCI Controllers and Peripherals	162

PCI References	163

Checklist for PCI	164

Chapter 10	ATA and ATAPI	167

ATA Controller Requirements	168

ATAPI Peripheral General Requirements	171

Plug and Play for ATA Controllers and Peripherals	172

Power Management for ATA Devices	173

ATA and ATAPI References	174

Checklist for ATA and ATAPI	175

Chapter 11	SCSI	177

SCSI Host Adapter Requirements	178

SCSI Peripheral Requirements	180

Plug and Play for SCSI Host Adapters and Peripherals	181

Power Management for SCSI Devices	183

SCSI References	184

Checklist for SCSI	185

Chapter 12	PC Card	187

PC Card Basic Requirements	188

PC Card Socket Controller Requirements	188

Plug and Play Design for 16-bit PC Card Cards	191

Plug and Play Design for CardBus	194

PC 99 Requirements for PC Card	196

PC Card References	198

Checklist for PC Card	199

Part 4  Device Design Guidelines

Chapter 13	I/O Ports and Devices	205

System Requirements for I/O Ports and Devices	206

Serial Port Requirements	208

Parallel Port Requirements	210

Mouse Port and Peripheral Requirements	214

Keyboard Port and Peripheral Requirements	215

Game Controller Requirements	217

Wireless Component Requirements	218

Smart Card Requirements	221

PC 99 Design Features for Ports	224

References for I/O Ports and Devices	226

Checklist for I/O Ports and Devices	228

Chapter 14	Graphics Adapters	231

System Requirements for Graphics Adapters	232

Graphics Adapters Basic Features	234

PC 99 Design for Graphics Adapters	255

Graphics Adapters References	261

Checklist for Graphics Adapters	261

Chapter 15	Video and Broadcast Components	265

System Requirements for Video and Broadcast Components	266

Basic Features for Video and Broadcast Components	268

MPEG-2 Video Playback Requirements	274

DVD-Video Playback Requirements	277

Video Input and Capture Requirements	280

Analog TV Tuner/Decoder and VBI Capture Requirements	282

Digital Broadcast TV Requirements	283

PC 99 Design for Video and Broadcast Components	287

Video and Broadcast Component References	289

Checklist for Video and Broadcast Components	290

Chapter 16	Monitors	295

Monitor Basic Features	296

Desktop Monitor Requirements	297

Entertainment Monitor Requirements	299

Plug and Play Design for Monitors	300

Power Management for Monitors	301

Monitors References	302

Checklist for Monitors	302

Chapter 17	Audio Components	305

Basic Audio Requirements	306

Advanced Audio Recommendations	311

PC 99 Design for Audio	315

Audio References	320

Checklist for Audio Components	321

Chapter 18	Storage and Related Peripherals	323

Storage Controller and Peripherals Basic Features	324

Floppy Disk Controller and Drive	326

Hard Disk Drives	327

CD Devices	328

Rewritable Optical ATAPI Devices	330

DVD Devices	331

PC 99 Design for Storage Components	333

Storage References and Resources	337

Checklist for Storage and Related Peripherals	338

Chapter 19	Modems	343

System Requirements for Modems	342

Modem Basic Features	342

Basic Modem Performance	354

Driver-based Modem Guidelines	356

PC 99 Design for Modems	359

Modem References	363

Checklist for Modems	364

Chapter 20	Network Communications	367

System Requirements for Network Communications	368

Network Adapter Requirements	369

ISDN Requirements	373

Cable Modem Requirements	376

ATM Adapter Requirements	378

ADSL Requirements	382

IrDA Requirements for Network Communications	384

Home Networking Requirements	385

PC 99 Design for Network Communications	387

Network Communications References	391

Checklist for Network Communications	392

Chapter 21	Printers	397

Basic Printer Features	398

PC 99 Printer Design	400

Printer References	403

Checklist for Printers	403

Chapter 22	Digital Still Image Peripherals	405

Digital Still Image Devices Basic Features	406

PC 99 Design for Digital Still Image Devices	410

Digital Still Image Device References	413

Checklist for Digital Still Image Devices	413

Appendixes

Appendix A	Icons	417

Appendix B	Device Identifiers	319

Plug and Play Vendor and Device IDs	420

Generic Windows Device IDs	421

Appendix C	Accessibility	435

Introduction	436

Visual Displays and Indicators	439

Sound	440

Manipulation and Physical Design	441

Input and Controls	443

Labeling	446

Documentation	447

Accessibility Recommendations for PC Design	448

Accessibility for PC Card	449

Accessibility Guidelines for Input Components	449

Accessibility Guidelines for Display Monitors	452

Accessibility Guidelines for Audio Components	453

Accessibility Guidelines for Storage Devices	454

Accessibility Guidelines for Printers	457

Accessibility References and Resources	459

Appendix D	Legacy Support	463

Fixed ISA Interrupts	464

Legacy ISA DMA Assignments	465

Legacy ISA I/O Address Assignments	465

Plug and Play ISA System Requirements	467

Plug and Play ISA Device Requirements	468

Appendix E	PC 99 Master Checklist	473

Glossary			509

Index		527