.BS

\l'6i'

.BE

.deTP

'sp

.)K

.af;P \\gP

.afP 1

.nr;P \\nP

.afP \\g(;P

.af;P 1

.ie\\n(Pv \{\

.ie(\\n(Pv=1)&(\\n(;P>1) 'sp 2

.el\{\

.ce

.ul

PRIVATE

.sp\} \}

.el'sp 2

.if!\\n(;P-1 .if \\nN 'sp

.if!\\n(;P-1 .if \\n(:S .tl \\*(}t

.if!\\n(;P-1 .if !\\nN .tl \\*(}t

.if\\n(;P-1 .ie \w'\\*(]n' .tl '\\*(]n - \\nP'''

.el.tl \\*(}t

\l'6i'

.sp

..

.nr Cl 4

.nr Hs 2

.nr Hb 3

.ds HF 3 3 3 3 3 3 3

.ds HP 12 11 10 10 10 10 10

.nr Ej 1

.PH "'Novell, Inc.'Compression for Wide Area Networks''"

.PF "'Introduction'' \\\\nP'"

.H 1 "Introduction

This short paper will present a number of compression options and

alternatives.  It is hoped that it will provide ample information to

choose an appropriate compression algorithm and method for

implementation in the Novell Wide Area Networking product.

It is desired to perform some type of compression on the data to be

transmitted to a remote site to increase the apparent bandwidth of

the (probably) low-speed link.  This compression can take many

forms, and can be applied at one or more layers of a networking

protocol.  In this paper, implementation at the data link layer

will be explored.

In selecting a compression algorithm, it is necessary to consider the

following items:

.BL

.LI

Compression ratio

.br

This is the ratio of the data emitted from the compression routine

versus the data received by the routine.  For example, 300,000 bytes

received and 100,000 bytes transmitted would be a 3:1 compression ratio.

.LI

Speed

.br

The algorithm's speed must be considered,

especially when Novell protocols are involved.

.LI

Size of memory required to implement the protocol

.br

The amount of dictionary space required in RAM for each protocol.

.LE

.H 2 "Description of Method"

To test the various compression methods, some representative

data was required.  This data was obtained from a public domain

collection of binary, text and image files, collected by T.C. Bell

and J.G. Cleary.  Collectively, these files are referred to as the

Calgary/Canterbury text compression corpus.  This corpus is used

in the book:

.in 1i

Bell, T.C., Cleary, J.G. and Witten, I.H. Text compression.

Prentice Hall, Englewood Cliffs, NJ, 1990

.in 0

and in the survey paper

.in 1i

Bell, T.C., Witten, I.H. and Cleary, J.G. "Modeling for text

compression," Computing Surveys 21(4): 557-591; December 1989

.in 0

The corpus is used to evaluate the practical performance of various

text compression schemes.  Several other researchers are now using the

corpus to evaluate text compression schemes.

Nine different types of text are represented, and to confirm that

the performance of schemes is consistent for any given type, many of

the types have more than one representative.  Normal English,

both fiction and non-fiction, is represented by two books and

papers (labeled book1, book2, paper1, paper2, paper3, paper4,

paper5, paper6).

More unusual styles of English writing are found in a bibliography

(bib) and a batch of unedited news articles (news). Three computer

programs represent artificial languages (progc, progl, progp). A

transcript of a terminal session (trans) is included to indicate

the increase in speed that could be achieved by applying compression

to a slow line to a terminal.  All of the files mentioned so

far use ASCII encoding.  Some non-ASCII files are also

included: two files of executable code (obj1, obj2), some

geophysical data (geo), and a bit-map black and white

picture (pic).  The file geo is particularly difficult to

compress because it contains a wide range of data values,

while the file pic is highly compressible because of large

amounts of white space in the picture, represented by long

runs of zeros.

More details of the individual texts are given in the book

mentioned above. Both book and paper give the results of

compression experiments on these texts.

The corpus itself constitutes files bib, book1, book2, geo, news,

obj1, obj2, paper1, paper2, paper3, paper4, paper5,

paper6, pic, progc, progl, progp and trans.  (The book and paper

above do not give results for files paper3, paper4,

paper5 or paper6.)

.TB "Input File descriptions"

.TS

box, center;

l | l | l | l | l

l | l | n | n | n.

Filename	Description	Lines	Words	Characters

_

bib	Bibliographic files (refer format)	6280	19274	111261

book1	Hardy: Far from the madding crowd	16622	141274	768771

book2	Witten: Principles of computer speech	15634	101221	610856

geo	Geophysical data	18	617	102400

news	News batch file	10059	53939	377109

obj1	Compiled code for Vax: compilation of progp	87	495	21504

obj2	T{

Compiled code for Apple Macintosh:

Knowledge support system

T}	1213	4600	246814

paper1	T{

Witten, Neal and Cleary:

Arithmetic coding for data compression

T}	1250	8512	53161

paper2	Witten: Computer (in)security	1731	13829	82199

paper3	Witten: In search of "autonomy"	1100	7219	46526

paper4	Cleary: Programming by example revisited	294	2166	13286

paper5	Cleary: A logical implementation of arithmetic	320	2099	11954

paper6	T{

Cleary: Compact hash tables using

bidirectional linear probing

T}	1019	6753	38105

pic	T{

Picture number 5 from the CCITT

Facsimile test files (text + drawings)

T}	0	49	513216

progc	C source code: compress version 4.0	1487	6313	39611

progl	Lisp source code: system software	2244	9235	71646

progp	T{

Pascal source code: prediction by partial

matching evaluation program

T}	1966	4847	49379

trans	Transcript of a session on a terminal	2737	9255	93695

Total		64061	391697	3251493

.TE

These files, on their own, do not correctly represent traffic

between routers.  They do, however, give a good base indication of

common types of files that are likely to occur.  To more

closely represent real traffic, a separate program was

written to combine all 20 of these files into a single data

stream, with PPP (Point-to-Point Protocol);

IPX (Internetwork Packet eXchange); and NCP

(Netware Core Protocol) headers prepended to the data.  Data from

each of the 20 files was read, 512 bytes at a time, until the

data was exhausted.  The 20 separate data streams were

linear multiplexed into a single data stream, of 3,531,333 bytes

(including headers).  For this (contrived) example,

the IPX and NCP overhead was about 8%.  The program in the

following figure produced this data stream.

.FG "Makepack.c"

.ft CW

.nf

#include <stdio.h>

#include "ipx.h"

#define NUMHOSTS 18

struct NCPPACK Hosts[NUMHOSTS];

char *Files[NUMHOSTS] = {

"../files/bib","../files/book1","../files/book2","../files/geo",

"../files/news","../files/obj1","../files/obj2","../files/paper1",

"../files/paper2","../files/paper3","../files/paper4","../files/paper5",

"../files/paper6","../files/pic","../files/progc","../files/progl",

"../files/progp","../files/trans"};

unsigned long Filepos[NUMHOSTS];

main(argc,argv)

int argc;

char **argv;

{

int x,y,done;

FILE *fo;

struct ipx *ipx;

struct ncp *ncp;

if (argc < 2) {

fprintf(stderr,"Usage: %s outfile\n",argv[0]);

fprintf(stderr,"Builds an output file with PPP/IPX/NCP headers 		from the standard\n");

fprintf(stderr,"compression benchmark test\n");

fprintf(stderr,"Version 1.00  Tue Nov 10 13:34:02 1992      		Dave Rand\n");

exit(1);

}

for (x=0; x< NUMHOSTS; x++) {

ipx = &Hosts[x].ipx;

ipx->ipx_sum = 0xffff;

ipx->ipx_len = 512 + sizeof(struct ncp) + sizeof(struct ipx);

ipx->ipx_tc = 0;

ipx->ipx_pt = NW_PROTO_NCP;

ipx->ipx_dna.x_net = rand() + (rand() << 16);

for (y=0; y<6; y++)

ipx->ipx_dna.x_node[y] = rand();

ipx->ipx_dna.x_sock = NW_SOCK_FS;

ipx->ipx_sna.x_net = rand() + (rand() << 16);

for (y=0; y<6; y++)

ipx->ipx_sna.x_node[y] = rand();

ipx->ipx_sna.x_sock = NW_SOCK_FS;

ncp = &Hosts[x].ncp;

ncp->ncp_op = NCP_REPLY;

ncp->ncp_seq = 0;

ncp->ncp_conn = rand();

ncp->ncp_task = rand();

Filepos[x] = 0L;

}

fo = fopen(argv[1],"wb");

if (fo == NULL) {

fprintf(stderr,"Can't open file %s for output!\n",argv[1]);

perror(argv[0]);

exit(2);

}

done = 0;

while (done != NUMHOSTS) {

done = 0;

for (x=0; x < NUMHOSTS; x++) {

if (Filepos[x] == -1L) {

done++;

} else {

packit(Files[x],&Filepos[x],&Hosts[x],fo);

}

}

}

fclose(fo);

}

unsigned char buf[512];

packit(file,pos,host,fo)

char *file;

unsigned long *pos;

struct NCPPACK *host;

FILE *fo;

{

FILE *fi;

int size;

fi = fopen(file,"rb");

if (fi == NULL) {

fprintf(stderr,"Can't open %s - continuing\n",file);

*pos = -1L;

return(1);

}

fseek(fi,*pos,0);

size = fread(buf,1,512,fi);

if (size != 512)

*pos = -1L;

else

*pos = ftell(fi);

fprintf(fo,"\377\003");

fwrite(host,sizeof(struct NCPPACK),1,fo);

fwrite(buf,size,1,fo);

host->ncp.ncp_seq++;

fclose(fi);

}

.R

.fi

.PF "'Test Results'' \\\\nP'"

.H 1 "Test Results"

The results of running the various compress programs are

presented in the following table.

.TB "Test results"

.TS

box,center;

l | l | l | l | l | l | l | l

l | l | n | n | n | n | n | n.

Compression Program	Method	Max ratio	Output size	Ratio	Time	K/sec	RGF

_

HPACKA 0.75c0	LZA		1272999	2.774	307	11.50	4146

Info-ZIP 1.9	LZSSA	905	1297993	2.721	90	39.24	14422

Gandalf FZA 	FZA		1310367	2.695

ARJ 2.30NG	LZSSA	1658	1313249	2.689	58	60.89	22642

LHA 2.13	LZAH	1883	1470858	2.401	66	53.51	22286

AR002	LZAH	1859	1470865	2.401	176	20.06	8357

HYPACK 2.5	LZAH	23810	1528534	2.310	47	75.13	32522

PKZIP 1.10	LZSF	519	1549210	2.279	53	66.63	29231

Unix COMPRESS	LZW	550	1580342	2.235

LZ	LZH	48	1641731	2.151	274	12.89	5992

DWC A5.01	LZW	532	1747822	2.020	11	321.03	158892

UNIX ARC	LZW	541	1872769	1.886

PKPAK 3.61	LZW	541	1912705	1.846	19	185.86	100668

LARC 3.33	LZSS	8	1914044	1.845	84	42.04	22786

MDCD 1.00	LZW	515	1917596	1.842	35	100.90	54788

ZOO 2.10	LZW	503	1917643	1.841	80	44.14	23971

SQZ 1.08.2	LZW		1937345	1.823	270	13.08	7175

STAC 3.1 /p255	LZS	30	1949600	1.811	22	160.52	88618

Sea ARC	LZW	3003	2024142	1.745

PRED 1.02	Prediction	8	2117640	1.668	5	706.27	423728

V.42bis (12 bit)	LZW	471	2274812	1.552	65	54.39	34997

STAC 3.1 /p0	LZS	30	2314985	1.525	13	271.64	178076

NSWP3 1.020	RLE-Huffman	336	2339012	1.510	46	76.77	50848

SPLAY 1.00	Splay-tree	8	2426472	1.455	71	49.74	34176

RLE 1.00	RLE-only	86	3157471	1.118	5	706.27	631494

Original input file			3531333	1.000

.TE

Compression program is the program used to evaluate the particular

compression method.  Notice that the same algorithm implemented by

different programs can produce different results.  This

usually reflects the programmer's choice of string search algorithm.

Method is the generic compression method applied to the input file.

Max ratio is the best-case compression ratio of the algorithm

expressed. It was determined by compressing a 1,000,000 byte

file filled with hex 00 characters.

Output size is the total output file size, including any

header information required to correctly decompress the file.

Time is the time required to compress the given file. In all

cases, decompression time was significantly better than

compression time, and is therefore not given in this table.

K/sec is the Kilobytes per second rate of the algorithm, as

seen from the input side (the ratio of the input file size

and the execution time).

RGF is the Rand Goodness Factor for the particular algorithm.

Higher numbers are better, and (coincidentally)

illustrate the expected bytes-per-second of output bandwidth

achievable with each implementation. For example, a

RGF of 4,146 would be an excellent choice for a 19,200

bps async modem channel, but not very effective on a T1

(throughput would be lower than without compression,

on this test data). The RGF is not a cure-all number; the

compression ratio, the destination link bandwidth, and

the input data set must be considered.

All of these algorithms, in their current form

(most of them are implemented in C or Pascal) are capable of

performing well on a <64Kbps link. The slower

algorithms (LHA) will show an adequate-to-very good

compression ratio.  Faster links are a problem, however.

Few of the algorithms in their current form are adequate

for medium to high speed links (>1Mbps, <10Mbps).

.PF "'Link issues'' \\\\nP'"

.H 1 "Link issues

For any of these compression algorithms to be implemented, a

reliable link between the source and destination

must be implemented.  This is required due to the

fact that even a one bit change in the compressed data stream

may affect hundreds or thousands of bytes in the

decompressed data stream.

There are many forms of reliable data delivery systems

available.  I suggest that the best one for the task at hand is

the LAPB (Link Access Protocol B) standard.  This

protocol has low overhead, known robustness, and is quite easy

to implement.  It is also the protocol under review by the

PPP compression group - though not accepted as a

standard at this time.

In order to maximize the bandwidth of the link, and to

allow for dynamic and transparent on-demand allocation of

additional dial-up links (including load sharing), the

upper-level packet structure will NOT be maintained through

to the data link level.  Since a packet may (potentially)

compress to just a few bits, it is worthwhile to re-packetize

the data. This offers better line utilization

(due to larger 2K packets), lower per-packet overhead, with

no increase in latency.

By observing the `backlog' of packets outstanding to

send on a link, line utilization figures can be obtained.  When

the line utilization is at (for example) 150%, a new link

can be established to the target system.  As more and more

links are established to the target system, the bandwidth

increases linearly.  By watching the highest-numbered line

(most recently established) traffic, that link can be

closed when the traffic through it falls below some threshold

level.  Notice as well that latency is reduced somewhat as

additional links are added.  Links that are dropped due to

line failure need not be re-established, unless they are

the only link to the target system.  Outbound traffic is always

sent starting at the lowest numbered line (least

recently established), and traffic is sent through additional lines

only if warranted.

The following figure shows a single link in action

.FG "Data Compressor Diagram

(not included)

As can be seen, the data compressor resides in the TSM.

This allows the TSM to open and establish additional

links to the target system by connecting to additional WHSM's.

Data will be sent to the compressor in the form of a

two-byte (network order) length ( of data in packet), the

contents of the packet, and a CRC-16 as found in RFC-1331. The

cost of computing this CRC is less than 0.5

milliseconds for a 1.5K input packet.  The addition of the

CRC-16 permits the compress/decompress pair to know

when they are out of synchronization, and force a

reset operation. It also allows for errors that may occur between

the WHSM and the TSM.

LAPB will be performed with CRC-32, as found in the

LAPB specification.  This ensures the maximum reliability

in data transmission between source and destination, with a very

slight (<0.1 %) increase in data size.  In the case

of synchronous data transmission, it will also offer faster

throughput due to software calculation of the CRC.  In

the current hardware, it will provide almost a 2% increase in speed.

.H 2 "CRC 16/CRC 32 Computation Overhead"

CRC 16 can also be referred to as the HDLC FCS, or more

properly, CRC-CCITT.  As described in RFC-1331, this

expresses the polynomial (omitted).  CRC 16 is useful for catching

all single bit, all two bit, most

burst errors of less than 16 bits, and some larger

errors in data streams of less than 32,767 bits in length (4K byte

frames), with a failure rate of 0.0015%.  Multi-bit

errors drastically reduce its effectiveness.

CRC 32 catches all single bit, two bit, and burst errors

less than 31 bits. In packets of less than 2,147,483,648 bits

(256 Mbytes), it catches larger errors with a failure

rate of  0.0000000023%. It expresses the polynomial (omitted).

CRC 32 is used commonly in Ethernet applications, and (while it

looks more complex) is just as easy to compute as CRC 16.

To compute CRC 16, one initializes a table of possible

results of each of the 256 possible input states, initializes a

variable to 0xFFFF, and iterates over the data with the following code:

.ft CW

fcs = (fcs >> 8) ^ fcstab[(fcs ^ DATA_BYTE) & 0xff];

.R

When all of the data has been fed through the CRC generator, the

variable fcs will contain the CRC 16 value for the code. It is normal

practice to complement this, and append it to the data. In the

case of CRC 16, when this is done the appended (complemented) CRC

can also be fed through the CRC generator. If the result of this is

0xF0B8, the frame is good.  The CRC can also be compared to the CRC

in the stream, and not run through the CRC generator (if desired).

An implementation of this in 8086 assembler would be as follows:

.ft CW

.nf

; AL has new byte to be computed

;

mov	cx,fcs		; get current FCS value

mov	bl,cl		; get lsb of fcs

xor	bl,al		; xor in current data byte

xor	bh,bh		; value is in the range 00-FF

shl	bx,1		; multiply bx by 2 to get word index

mov	ax,fcstab[bx]	; get new value from table

xor	al,ch		; xor old fcs value

mov	fcs,ax		; save current fcs

.R

.fi

In theory, this code should run in (1+1+1+1+1+1+1 = 7) 7 clocks on a

486, for a throughput of 4.7 megabytes per second (or about 325

microseconds for a 1.5Kbyte packet) on a 33 MHz 486 system.

On an 8088, this routine takes (19 +2+2+2+2+22+19 = 68) 68 clocks,

for a throughput of 70 Kbytes/sec (or 22 milliseconds for a 1.5Kbytes

packet). Note that in the case of the 486, the throughput

exceeds that of the ISA bus. Of course, this can be

optimized - placing the FCS in a register will raise the

throughput to 6.6 Meg/sec; using 32 bit instructions will

raise throughput to 8.25 Meg/sec.

To compute CRC 32, one initializes a table of possible

results of each of the 256 possible input states, initializes a

variable to 0xFFFFFFFF, and iterates over the data with the

following code:

.ft CW

fcs = (fcs >> 8) ^ fcstab32[(fcs ^ DATA_BYTE) & 0xff];

.R

When all the data has been fed through the CRC generator,

the variable fcs will contain the CRC 32 value for the

code. It is normal practice to complement this, and append it

to the data. In the case of CRC 32, when this is done

the appended (complemented) CRC can also be fed through the

CRC generator. If the result of this is 0xDEBB20E3, the frame

is good.  The CRC can also be compared to the CRC in the stream,

and not run through the CRC generator (if desired).

An implementation of this in 8086 assembler would be as follows:

.ft CW

.nf

; AL has new byte to be computed

;

mov	cx,fcs		; get lsw of current FCS value

mov	dx,fcs+2		; get msw of FCS

mov	bl,cl		; get lsb of fcs

mov	cl,ch		; shift FCS right 8 bits

mov	ch,dl		; dh has msb

xor	bl,al		; xor in current data byte

xor	bh,bh		; value is in the range 00-FF

shl	bx,1		; multiply by 4 as

shl	bx,1		; each entry is 1 doubleword

xor	cx,fcstab[bx]	; xor table with old fcs value

mov	fcs,cx		; save current fcs

mov	ax,fcstab+2[bx] ; get msw of fcstable

xor	al,dh		; xor with old fcs value

mov	fcs+2,ax		; save result

.R

.fi

In theory, this code should run in (1+1+1+1+1+1+1+1+1+1+1+1+1+1 = 14)

14 clocks on a 486, for a throughput of 2.4 megabytes per second

(or about 651 microseconds for a 1.5Kbyte packet) on a 33 MHz

486 system. On an 8088, this routine takes

(19+19+2+2+2+2+2+2+2+22+19+22+2+19 = 136) 136 clocks, for a

throughput of 35 Kbytes/sec (or 44 milliseconds for a 1.5Kbytes

packet).  Of course, this can be optimized - placing the FCS in a

register will raise the throughput to 3.3 Meg/sec;

using 32 bit instructions will raise throughput to 8.25 Meg/sec.

.PF "'Recommendations'' \\\\nP'"

.H 1 "Recommendations

I suggest that at least three algorithms be used to perform the

compression.  The choice of the algorithm should be

made based on the aggregate speed of the data link to the

target system, with override capability by a configuration

option.  The following table illustrates the recommendation.

.TB "Compression recommentations"

.TS

box,center;

l | l | l | l.

Line speed	Compression algorithm	Compression ratio	Effective speed / Max speed

_

<= 64Kbps	LZSSA	2.721	174 Kbps / 58 Mbps

<= 2 Mbps	LZW (max cw 16 bits)	2.235	4.5 Mbps / 1100 Mbps

<=10 Mbps	RLE	1.668	16.7 Mbps /  80 Mbps

.TE

The maximum speed is the theoretical maximum

compression ratio for the algorithms specified.  It is extremely

unlikely that these speeds would ever be seen in practice.

The effective speed shows the speed that could be

obtained running at the maximum specified line speed,

with the sample data stream detailed in the method section

of this document.  Real, effective speeds will (of course)

depend on the data set. Fully random data will achieve

effective line speeds of approximately 90% of available

bandwidth (ignoring hardware limitations).

.PF "'Background Material'' \\\\nP'"

.H 1 "Background Material

The following material was written by Haruhiko Okumura, and

gives some background on each of the various compression algorithms.

.ce

Data Compression Algorithms of LARC and LHarc

.H 2 Introduction

In the spring of 1988, I wrote a very simple data

compression program named LZSS in C language, and uploaded

it to the Science SIG (forum) of PC-VAN, Japan's biggest

personal computer network.

That program was based on Storer and Szymanski's

slightly modified version of one of Lempel and Ziv's

algorithms.  Despite its simplicity, for most files its

compression outperformed the archivers then widely used.

Kazuhiko Miki rewrote my LZSS in Turbo Pascal and

assembly language, and soon made it evolve into a complete

archiver, which he named LARC.

The first versions of LZSS and LARC were rather slow.

So I rewrote my LZSS using a binary tree, and so did

Miki.  Although LARC's encoding was slower than the

fastest archiver available, its decoding was quite fast, and

its algorithm was so simple that even self-extracting

files (compressed files plus decoder) it created were usually

smaller than non-self-extracting files from other archivers.

Soon many hobby programmers joined the archiver project

at the forum. Very many suggestions were made, and

LARC was revised again and again. By the summer of 1988,

LARC's speed and compression have improved so

much that LARC-compressed programs were beginning to be

uploaded in many forums of PC-VAN and other

networks.

In that summer I wrote another program, LZARI, which

combined the LZSS algorithm with adaptive arithmetic

compression.  Although it was slower than LZSS, its

compression performance was amazing. Miki, the author of

LARC, uploaded LZARI to NIFTY-Serve, another big

information network in Japan.  In NIFTY-Serve, Haruyasu

Yoshizaki replaced LZARI's adaptive arithmetic

coding with a version of adaptive Huffman coding to increase

speed.  Based on this algorithm, which he called LZHUF, he

developed yet another archiver, LHarc.

In what follows, I will review several of these

algorithms and supply simplified codes in C language.

.H 2 "Simple coding methods

Replacing several (usually 8 or 4) "space"

characters by one "tab" character is a very primitive method for data

compression.  Another simple method is run-length coding, which

encodes the message "AAABBBBAACCCC"

into "3A4B2A4C", for example.

.H 3 "LZSS coding"

This scheme is initiated by Ziv and Lempel [1].  A

slightly modified version is described by Storer and Szymanski

[2].  An implementation using a binary tree is proposed by

Bell [3].  The algorithm is quite simple: Keep a ring

buffer, which initially contains "space" characters only.

Read several letters from the file to the buffer.  Then

search the buffer for the longest string that matches

the letters just read, and send its length and position in the

buffer.  If the buffer size is 4096 bytes, the position

can be encoded in 12 bits.  If we represent the match length in

four bits, the <position, length> pair is two bytes long.  If

the longest match is no more than two characters, then

we send just one character without encoding, and restart

the process with the next letter.  We must send one extra

bit each time to tell the decoder whether we are sending a

<position, length> pair or an unencoded character.

The accompanying file LZSS.C is a version of this algorithm.

This implementation uses multiple binary trees to

speed up the search for the longest match.  All the

programs in this article are written in draft-proposed ANSI C.  I

tested them with Turbo C 2.0.

.H 3 "LZW coding

This scheme was devised by Ziv and Lempel [4], and

modified by Welch [5]. The LZW coding has been adopted

by most of the existing archivers, such as ARC and PKZIP.  The

algorithm can be made relatively fast, and is

suitable for hardware implementation as well.

The algorithm can be outlined as follows: Prepare a

table that can contain several thousand items.  Initially register

in its 0th through 255th positions the usual 256 characters.

Read several letters from the file to be encoded, and

search the table for the longest match.  Suppose the

longest match is given by the string "ABC".  Send the position

of "ABC" in the table.  Read the next character from the

file.  If it is "D", then register a new string "ABCD" in the

table, and restart the process with the letter "D".  If the

table becomes full, discard the oldest item or, preferably,

the least used.  A Pascal program for this algorithm is

given in Storer's book [6].

.H 3 "Huffman coding"

Classical Huffman coding was invented by

Huffman [7].  A fairly readable account is given in Sedgewick [8].

Suppose the text to be encoded is "ABABACA", with

four A's, two B's, and a C.  We represent this situation as

follows:

.ft CW

.nf

4	2    1

|	|    |

A	B    C

.fi

.R

Combine the least frequent two characters into one,

resulting in the new frequency 2 + 1 = 3:

.ft CW

.nf

4	  3

|	 / \\

A	B   C

.fi

.R

Repeat the above step until the whole characters combine into a tree:

.ft CW

.nf

7

/ \\

/   3

/   / \\

A   B   C

.R

.fi

Start at the top ("root") of this encoding tree, and

travel to the character you want to encode.  If you go left, send a

"0"; otherwise send a "1".  Thus, "A" is encoded by "0", "B"

by "10", "C" by "11". Altogether, "ABABACA" will

be encoded into ten bits, "0100100110".

To decode this code, the decoder must know the encoding tree,

which must be sent separately.

A modification to this classical Huffman coding is the

adaptive, or dynamic, Huffman coding.  See, e.g., Gallager

[9].  In this method, the encoder and the decoder processes

the first letter of the text as if the frequency of each

character in the file were one, say.  After the first

letter has been processed, both parties increment the frequency of

that character by one.  For example, if the first letter is 'C',

then freq['C'] becomes two, whereas every other

frequencies are still one. Then the both parties

modify the encoding tree accordingly.  Then the second letter will

be encoded and decoded, and so on.

.H 3 "Arithmetic coding

The original concept of arithmetic coding is

proposed by P.  Elias.  An implementation in C language is described

by Witten and others [10].

Although the Huffman coding is optimal if each character

must be encoded into a fixed (integer) number of bits,

arithmetic coding wins if no such restriction is made. As

an example we shall encode "AABA" using arithmetic

coding.  For simplicity suppose we know beforehand that

the probabilities for "A" and "B" to appear in the text are

3/4 and 1/4, respectively.

Initially, consider an interval:

.ft CW

0 <=  x  < 1.

.R

Since the first character is "A" whose probability is 3/4,

we shrink the interval to the lower 3/4:

.ft CW

0 <=  x  < 3/4.

.R

The next character is "A" again, so we take the lower 3/4:

.ft CW

0 <=  x  < 9/16.

.R

Next comes "B" whose probability is 1/4, so we take the upper 1/4:

.ft CW

27/64 <=  x  < 9/16,

.R

because "B" is the second element in our alphabet, {A, B}.

The last character is "A" and the interval is

.ft CW

27/64 <=  x  < 135/256,

.R

which can be written in binary notation

.ft CW

0.011011 <= x  < 0.10000111.

.R

Choose from this interval any number that can be represented in

fewest bits, say 0.1, and send the bits to the right

of "0."; in this case we send only one bit, "1".  Thus we have

encoded four letters into one bit! With the Huffman

coding, four letters could not be encoded into less than four bits.

To decode the code "1", we just reverse the process: First,

we supply the "0." to the right of the received code "1",

resulting in "0.1" in binary notation, or 1/2.  Since this

number is in the first 3/4 of the initial interval 0 <= x < 1,

the first character must be "A".  Shrink the interval into the

lower 3/4.  In this new interval, the number 1/2 lies in

the lower 3/4 part, so the second character is again "A", and so

on.  The number of letters in the original file must

be sent separately (or a special 'EOF' character must be

appended at the end of the file).

The algorithm described above requires that both the

sender and receiver know the probability distribution for the

characters.  The adaptive version of the algorithm removes this

restriction by first supposing uniform or any

agreed-upon distribution of characters that approximates the

true distribution, and then updating the distribution

after each character is sent and received.

.H 3 "LZARI

In each step the LZSS algorithm sends either a

character or a <position, length> pair.  Among these, perhaps

character "e" appears more frequently than "x",

and a <position, length> pair of length 3 might be commoner than

one of length 18, say.  Thus, if we encode the more frequent

in fewer bits and the less frequent in more bits, the

total length of the encoded text will be diminished.

This consideration suggests that we use Huffman or arithmetic

coding, preferably of adaptive kind, along with LZSS.

This is easier said than done, because there are many

possible <position, length> combinations.  Adaptive

compression must keep running statistics of frequency distribution.

Too many items make statistics unreliable.

What follows is not even an approximate solution to the

problem posed above, but anyway this was what I did in

the summer of 1988.

I extended the character set from 256 to three-hundred or

so in size, and let characters 0 through 255 be the usual

8-bit characters, whereas characters 253 + n represent that

what follows is a position of string of length n, where n

= 3, 4 , ....  These extended set of characters will be

encoded with adaptive arithmetic compression. I also observed

that longest-match strings tend to be the ones that were

read relatively recently.  Therefore, recent positions should

be encoded into fewer bits.  Since 4096 positions are too

many to encode adaptively, I fixed the probability

distribution of the positions "by hand." The distribution

function given in the accompanying LZARI.C is rather

tentative; it is not based on thorough experimentation.  In

retrospect, I could encode adaptively the most significant

6 bits, say, or perhaps by some more ingenious method adapt

the parameters of the distribution function to the

running statistics.

At any rate, the present version of LZARI treats the

positions rather separately, so that the overall compression is

by no means optimal. Furthermore, the string length threshold

above which strings are coded into <position,

length> pairs is fixed, but logically its value must

change according to the length of the <position, length> pair we

would get.

.H 3 "LZHUF

LZHUF, the algorithm of Haruyasu Yoshizaki's

archiver LHarc, replaces LZARI's adaptive arithmetic coding with

adaptive Huffman.  LZHUF encodes the most significant 6 bits

of the position in its 4096-byte buffer by table

lookup.  More recent, and hence more probable, positions are

coded in less bits.  On the other hand, the remaining

6 bits are sent verbatim.  Because Huffman coding encodes

each letter into a fixed number of bits, table lookup can

be easily implemented. Though theoretically Huffman cannot

exceed arithmetic compression, the difference is very

slight, and LZHUF is fairly fast.

The accompanying file LZHUF.C was written by Yoshizaki.

I translated the comments into English and made a

few trivial changes to make it conform to the ANSI C standard.

.H 2 "References

.nf

[1] J. Ziv and A. Lempel, IEEE Trans. IT-23, 337-343 (1977).

[2] J. A. Storer and T. G. Szymanski, J. ACM, 29, 928-951 (1982).

[3] T. C. Bell, IEEE Trans. COM-34, 1176-1182 (1986).

[4] J. Ziv and A. Lempel, IEEE Trans. IT-24, 530-536 (1978).

[5] T. A. Welch, Computer, 17, No.6, 8-19 (1984).

[6] J. A. Storer, Data Compression: Methods and Theory (Computer Science Press, 1988).

[7] D. A. Huffman, Porch IRE 40, 1098-1101 (1952).

[8] R. Sedgewick, Algorithms, 2nd ed. (Addison-Wesley, 1988).

[9] R. G. Gallager, IEEE Trans. IT-24, 668-674 (1978).

[10] I. E. Witten, R. M. Neal, and J. G. Cleary, Commun. ACM 30, 520-540 (1987).

.fi

.PF "'Legal Issues'' \\\\nP'"

.H 1 "Legal Issues

Regretfully, no compression algorithms are free of patent

contention. The following items come from the

comp.compression frequently asked questions list. I make

no claims as to the validity of any of the patent issues,

nor of any of the implementations under investigation.

.BL

.LI

Run length encoding

.br

Tsukiyama has two patents on run length encoding: 4,586,027

and 4,872,009 granted in 1986 and 1989

respectively. The first one covers run length encoding in its

most primitive form: a length byte followed by the

repeated byte. The second patent covers the 'invention' of

limiting the run length to 16 bytes and thus the encoding

of the length on 4 bits.  Here is the start of claim 1 of

patent 4,872,009, just for pleasure:

1. A method of transforming an input data string comprising a

plurality of data bytes, said plurality

including portions of a plurality of consecutive data

bytes identical to one another, wherein said data bytes

may be of a plurality of types, each type representing

different information, said method comprising the

steps of: [...]

.LI

LZ77

.br

The Gibson & Graybill patent 5,049,881 covers the

LZRW1 algorithm previously discovered by Ross Williams.

Claims 4 and 12 are very general and could be interpreted

as applying to any LZ algorithm using hashing

(including all variants of LZ78):

4. A compression method for compressing a stream of

input data into a compressed stream of output data

based on a minimum number of characters in each input

data string to be compressed, said compression

method comprising the creation of a hash table,

hashing each occurrence of a string of input data and

subsequently searching for identical strings of input data

and if such an identical string of input data is

located whose string size is at least equal to the minimum

compression size selected, compressing the

second and all subsequent occurrences of such identical

string of data, if a string of data is located which

does not match to a previously compressed string of data,

storing such data as uncompressed data, and for

each input strings after each hash is used to find a

possible previous match location of the string, the

location of the string is stored in the hash table,

thereby using the previously processed data to act as a

compression dictionary.

Claim 12 is identical, with 'method' replaced

with 'apparatus'. Since the 'minimal compression size' can be as small

as 2, the claim could cover any dictionary technique of the

LZ family. However the text of the patent and the other

claims make clear that the patent should cover the LZRW1

algorithm only.

The following papers, published before the patent was

filed, describe applications of hashing to LZ77 compression:

Brent, R.P.  "A Linear Algorithm for Data Compression",

Australian Computer Journal, Vol.19, No.2 (May

1987), p.64.

Bell, T. "Longest match string searching for Ziv-Lempel

compression" Res. Rept. 6/89, Dept. of Computer

Science, Univ. of Canterbury, New Zealand (Feb 89).

Phil Katz, author of pkzip, also has a patent on

LZ77 (5,051,745) but the claims only apply to sorted hash tables,

and when the hash table is substantially smaller than the window size.

Robert Jung, author of 'arj', has recently been granted

patent 5,140,321 for one variation of LZ77 with hashing.

This patent covers the LZRW3-A algorithm, also previously

discovered by Ross Williams. LZRW3-A was posted

on comp.compression on July 15, 1991. The patent was

filed two months later on Sept 4, 1991. (The US patent

system allows this because of the 'invention date' rule.)

Fiala and Greene obtained in 1990 a patent (4,906,991) on

all implementations of LZ77 using a tree data structure.

Claim 1 of the patent is much broader than the algorithms

published by Fiala and Greene in Comm.ACM, April

89. The patent covers the algorithm published by Rodeh

and Pratt in 1981 (J. of the ACM, vol 28, no 1, pp 16-24).

It also covers the algorithm previously patented by

Eastman-Lempel-Ziv (4,464,650), and the algorithms used in

lharc, lha and zoo.

IBM patented (5,001,478) the idea of combining a

history buffer (the LZ77 technique) and a lexicon (as in LZ78).

.LI

LZ78

.br

The LZW algorithm used in 'compress' is patented

by IBM (4,814,746)   and Unisys (4,558,302). It is also used in

the V.42bis compression  standard and in Postscript Level 2.

(Unisys sells the license to modem manufacturers for

a onetime $25,000 fee.) The IBM patent application was filed

three weeks before that of Unisys, but the US patent

office failed to recognize that they covered the same

algorithm. (The IBM patent is more general, but its claim 7 is

exactly LZW.)

AP coding is patented by Storer (4,876,541).

.LI

Other data compression algorithms

.br

IBM holds a patent on the Q-coder implementation of

arithmetic coding.  The arithmetic coding option of the

JPEG standard requires use of the patented algorithm.  No

JPEG-compatible method is possible without infringing

the patent, because what IBM actually claims rights to is

the underlying probability model (the heart of an

arithmetic coder).

Bacon has patented (4,612,532) some form of Markov modeling.

.LE

As can be seen from the above list, all the most popular

compression programs (compress, pkzip, zoo, lha, arj) are

now covered by patents. (This says nothing about the

validity of these patents.)

Here are some references on data compression patents. A

number of them are taken from the list maintained by

Michael Ernst <mernst@theory.lcs.mit.edu> in

mintaka.lcs.mit.edu:/mitlpf/ai/patent-list (or patent-list.Z).

4.464.650

.br

Apparatus and method for compressing data signals and

restoring the compressed data signals. Inventors Lempel,

Ziv, Cohn, Eastman. Assignees Sperry Corporation and AT&T

Bell Laboratories. Filed 8/10/81, granted 8/7/84

4,558,302

.br

High speed data compression and decompression apparatus and

method. Inventor Welch. Assignee Sperry

Corporation (now Unisys). Filed 6/20/83, granted 12/10/85. The

text for this patent can be ftped from

rusmv1.rus.uni-stuttgart.de (129.69.1.12) in

/info/comp.patents/US4558302.Z.

4,586,027

.br

Method and system for data compression and

restoration. Assignee Hitachi, inventor Tsukimaya et al. Filed

08/07/84, granted 04/29/86

4,612,532

.br

Inventor Bacon. Granted 9/1986

4,814,746

.br

Data compression method. Inventors Victor S. Miller,

Mark N. Wegman. Assignee IBM. Filed 8/11/86, granted

3/21/89. A previous application was filed on 6/1/83,

three weeks before the application by Welch (4,558,302)

4,872,009

.br

Method and apparatus for data compression and restoration.

Assignee Hitachi, inventor Tsukimaya et al. Filed

12/07/87, granted 10/03/89.

4,876,541

.br

Stem [sic] for dynamically compressing and decompressing

electronic data. Inventor James A. Storer. Assignee

Data Compression Corporation. Filed 10/15/87, granted 10/24/89

4,955,066

.br

Compressing and Decompressing Text Files.

Inventor Notenboom, L.A. Assignee Microsoft. Filed  10/13/89,

granted 09/04/90.

5,001,478

.br

Method of Encoding Compressed Data. Filed 12/28/89,

granted 03/19/91. Inventor Michael E. Nagy. Assignee IBM.

5,049,881

.br

Apparatus and method for very high data rate-compression

incorporating lossless data compression and expansion

utilizing a hashing technique. Inventors Dean K. Gibson,

Mark D. Graybill. Assignee Intersecting Concepts, Inc.

Filed 6/18/90, granted 9/17/91.

5,051,745

.br

String searcher, and compressor using same.

Inventor  Phillip W. Katz (author of pkzip). Filed  8/21/90, granted

9/24/91.

4,906,991

.br

Textual substitution data compression with

finite length search window. Inventors Fiala,E.R., and Greene,D.H.

Filed 4/29/1988, granted 3/6/1990. Assignee Xerox Corporation.

5,109,433

.br

Compressing and decompressing text files. Assignee Microsoft.

5,140,321

.br

Data compression/decompression method and apparatus.

Filed 9/4/91, granted 8/18/92. Inventor Robert Jung.

Assignee Prime Computer

Abbreviations for the compression method are:

Huffman - Dynamic Huffman coding; RLE - Run Length

Encoding; LZW - Lempel-Ziv-Welch dictionary coding;

LZH - Lempel-Ziv static Huffman coding; LZSF -

Lempel-Ziv with Shannon-Fano tree coding;

LZAH - Lempel-Ziv with Adaptive Huffman coding; LZA - Lempel-

Ziv with Arithmetic coding;

LZSS - Lempel-Ziv, as modified by Storer and Szymanski;

LZSSA - LZSS with

adaptive coding.

Time to run algorithm on an AST 486/33, excluding file I/O time.

.SK

.R

.fi

.BS

.BE

.deTP

'sp

.)K

.af;P \\gP

.afP 1

.nr;P \\nP

.afP \\g(;P

.af;P 1

.ie\\n(Pv \{\

.ie(\\n(Pv=1)&(\\n(;P>1) 'sp 2

.el\{\

.ce

.ul

PRIVATE

.sp\} \}

.el'sp 2

.if!\\n(;P-1 .if \\nN 'sp

.if!\\n(;P-1 .if \\n(:S .tl \\*(}t

.if!\\n(;P-1 .if !\\nN .tl \\*(}t

.if\\n(;P-1 .ie \w'\\*(]n' .tl '\\*(]n - \\nP'''

.el.tl \\*(}t

.sp

..

.PH

.SK

.TC 4 1 2 0

.SK

.PF

.rs

.sp 2i

.ns

.S 20

.B

.ad r

Data Compression for

.sp 10p

Wide Area Networks

.S

.R

.sp 2i

Revision 1.01   \n(mo/\n(dy/\n(yr

.sp 1.5i

Dave Rand

.br

Novell, Inc.

.br

San Jose, California

.sp 0.5i

.I