Release 10.1 of the Crynwr 3C509 Packet Driver

For the copyright, see COPYING.DOC.  In part it says that: Anyone

with a copy of the drivers may give it away, and the source code for

all modules must be available.

NO WARRANTY, see COPYING.DOC, but support for packet drivers is

available, see SUPPORT.DOC.

This release contains only the 3C509 packet driver.  Many more packet

drivers (including 3c501, 3c503, 3c505, 3c507, and 3c523) are

available in the complete packet driver release.  See HOWTOGET.IT for

availability.

Installation:

The packet driver has two parameters, one required and one optional.

Run the driver with one parameter, the software interrupt. A packet

driver needs to have a software interrupt assigned to it for other

programs to access it.  I recommend 0x7f or 0x7e.

The driver needs an initial ID port to access the 3C509 hardware. The

driver uses a default of 0x110 for the ID port.  If you have a

conflict at port 0x110 (very few machines do), the packet driver will

be unable to initialize the adapter, so try specifying a different

value as the second parameter.  ID port must be 0x100, 0x110, 0x120,

..., 0x1f0.

Notes:

The 3C509 is a good, fast, cheap board.  However, it requires close

cooperation between the driver and the adapter.  If your system has

high interrupt latency, this cooperation is disrupted.  Also, the

decisions made by different driver authors will affect whether a

driver/adapter combination will work in your system.

In particular, the bursty high packet rate typical of NFS

implementations causes the 3C509 to overrun.  You may have to reduce

the maximum length of packets sent to the 3C509 to transfer large

files.

This driver is ONLY supported and tested on the 3C509.  This driver

will be enhanced in the future to support the EISA-3C579 and MC-3C529

versions of the Etherlink III  family.  It currently contains code to

also support the EISA-3C579 card.  I have only been able to test

the driver on my EISA 486, where it doesn't work.  The problem shows

itself by corrupting the contents of packets.  But this could just be

my system (which is of dubious origin).  If you would like to beta

test this driver on a 3C579, please tell me how your trial went.

Please report problems.  I cannot guarantee that I will work on your

problem if you are not a Crynwr customer.  I CAN guarantee that I

will not work on it if you don't report it to me.  Written reports

are preferred to phone calls.

Russell Nelson

nelson@crynwr.com

315-268-1925 voice

315-268-9201 FAX

Crynwr Software, 11 Grant St., Potsdam, NY 13676