Packet Driver for BICC 4110-2 AT and MCA controllers

ISOLINK <packet_int_no> <options>

options: [defaults]

/I<hw-int-level> [2]

/P<I/O port address> [280]

/D<DMA channel #> [0]

/R<# recv buffs> <recv buff size> [16 256]

/T<# xmit buffs> <xmit buff size> [1 0]

/X

The /R option specifies the number of buffers the driver reserves for

the Lance chip.  If a buffer is too small, the arriving packet

overflows into the next buffer until the packet is received

completely.  If all buffers are full when a packet arrives, a MISS

error occurs and the packet is effectively lost.  Nothing can easily

done about this.  The error is indicated by incrementing error_in

count which can be displayed with a packet driver statistics program

(like FTP, Inc.  INET DEBUG or Russ Nelson's STAT).  The default

chosen may be to small...

The /X option instructs the driver to relocate itself and all buffers

into the first segment in extended memory (HMA - still addressible in

real mode).  The driver uses Microsoft's HIMEM.SYS to enable the

chip's A20 address line.  HIMEM.SYS is available from Microsoft (I

think it's automatically distributed with Windows/286) or from the

SIMTEL archives.  The A20 line is enabled once and left enabled

PERMANENTLY, a technique Microsoft explicitly discourages! According

to them certain applications rely on 1MB memory wrap.  I suggest to

avoid such applications anyway.  Specifying the /X option allocates

the HMA to the driver who does not necessarily use it most

efficiently; it is then unavailable to other programs (e.g.  Windows)

who could make better use of it. With the /X option the driver uses

all the remaining HMA for receive buffers (largest number of buffers

at least as long as requested).

The /T option specifies a number of transmit buffers.  If the outgoing

packet fits into a single buffer, it is copied and the Lance

instructed to send it out.  The network application resumes

immediately in this case.  If the packet does not fit, it is sent

directly from the user supplied address and the driver waits until it

has been sent out.

On the PS/2 the configured values for /I and /P replace the defaults

indicated, so that it does not make sense to specify them.

PROBLEMS

========

Still to be done:

- Errors are counted in a rather primitive manner

- terminating the driver does not reset the Lance chip and

does not free the HMA

- could one do useful things while waiting for the packet

to be transmitted?

I'm too busy on other things to chase a persistent bug in the

driver: when using NCSA telnet to a VAX 8530 running Ultrix and doing a

'ls -l /etc' the session hangs. Tracing shows the VAX keeps sending

packets from time to time, and the driver upcalls the handler routine

but nothing happens. Perhaps a packet lost/corrupted somewhere.

I'm afraid this may also happen in other circumstances.

Perhaps somebody will fix it some day. Alas, I probably won't!

Anyway, feel free to send comments or suggestions to rtb@cernvm.cern.ch