Release Notes - DC21X4 NDIS 3.x 4.0 and 5.0 drivers

Version v5.00

10-Nov-1998

This kit includes Release v5.00 of the DC21X4 NDIS 3.x, 4.0 and 5.0 drivers.

Changes from previous release:

==============================

- Added digital signature file and updated NETPCI.INF file to Win98 NDIS5 driver.

- Change all the Copyright and string notices from Digital to Intel.

- First NDIS5.0 driver release.

The NDIS5 driver symbols are built in the pdb format.

- Add support for Warm-Plug (PHY connection only in StandBy or Hybernate state)

of external MII PHY. In case that the SROM specifies this support, the driver

will not fail under non successfull MII PHY reset, but will only report

LinkFail indication to the wrapper.

- Fix problem to work with AUI/BNC medium when there is MII Phy

on the board. This problem appeared only from version 4.31 of the driver.

- Change the MII_DELAY value (minimum delay between two consecutive acceses to

the MII PHY) from 1usec to 2 usec because a problem found in ALPHA machines

running NT5.0 OS.

- Move the stopping of the adapter out from the ISR and set a timer to do it

in the "DeferredReset" routine (when a hardware error is detected).

- Set A new keyword: ProcessTransmitFirst - default disable. When this

keyword is set, the transmit packet ring will be processed before the

receive packet ring.

- Change a number of Transmit ring counters ans Indexes from UINT to INT

type to prevent errors (UINT will never be < 0 )

- Include hooks for NdisDbg utility support.

- Change the maximum number of MapRegisters from 64 to 128 for performance

improvements.

- Add a check to the MII connection medium on each Link status polling, to

workaround the MII Microlinear's problem with the Link Status sticky bit.

History:

========

Release V4.31

=============

- Add support for autosensing the size of the SROM (1Kbit or 4Kbit)

and reading/writing 4Kbit SROM.

- Serialize the Link indication to the wrapper after the driver initialization.

- Set LINK_HYSTERESIS bit (CSR6<25>) for 21143 symbol Phy boards for All

mediums (10 Mbps and 100 Mbps). This will check the Link presence

for a longer period.

- The Driver now supports enabling of Extended PCI commands,

(the commands new as of PCI spec 2.1), for those 2104x/2114x

devices which support the commands.

Enabling the commands may increase PCI or CardBus throughput,

depending on the system implementation.

As a default, the driver will enable the maximum possible number of

the three Extended PCI Commands,

Memory Write Invalidate (MWI);

Memory Read Line (MRL);

Memory Read Multiple (MRM);

after verifying that the system implementation (PCI bridge chipset

and BIOS) supports it.

The devices for which the commands will be enabled:

21140A rev >= 21 ,

21143 rev >= 20.

Due to incompatibility problems with certain PCI host bridges

and PCI-to-PCI bridges, the driver currently does not enable

the Memory Write Invalidate (MWI) and Memory Read Line (MRL) commands

simultaneously. The driver will only enable one of the two commands;

by default, MRL will be enabled.

The driver has 3 new keywords EXT_MWI, EXT_MRM and EXT_MRL.

These keyword enable and disable the operation of the

respective Extended PCI Commands, to allow user override; in no

case will an invalid combination be allowed.

- New OIDs to support DMI requirements.

- Fix the "Initialize Done" delay (V4.20) for the boards containing a BNC

implementation too. Set the Link timeout to 5 seconds.

- Fix the SROM Extended Block Type 3 (Mii PHY Block) Reset sequence field parsing.

The data for every element in the sequence must be in words.

- Change the delay for the reset sequence (at init time) to 5 Miliseconds.

- Fix the CSR13-15 values while working in MII mode; the old values (wrong)

caused a performance drop while working in Autosensing mode.

- Add support for the OID: OID_GEN_MAXIMUM_SEND_PACKETS .

- Fix a potential problem where a min buffer sent from the wrapper could be

accepted for transmision while no free descriptors were available.

Release V4.22

=============

- This release was the final release of the NT3.5 driver.

- Support the "Surprise removal of a PC Card device", which prevents any

spontaneous reboot, system stall or blue screen if the user accidentally

pulls the PC Card out of its socket.

- Fix the link interrupt masking on 100Mbps force mode on power-up. This problem

caused the driver to fail to get the 100Mbps link.

- Fix the access to the PCI configuration registers (for PCI mechanism 1) in

fullmac driver, which can cause power-up hangs in some machines in

Windows for workgroups environments (fullmac driver only).

Release V4.20 (beta):

=====================

- Fix the masking of Link iterrupts in the autosensing algorithm,

which caused a problem in the Link indication after reconnection,

at 100Mbps speed. The problem was present in 21143/QSI based boards

and 21142/Microlinear Cardbus based boards.

- Add private OIDs for SecureON (miniport only).

- Delay the "Initialize Done" indication until the NWAY process finishes

(specially for symbol PHY's based boards), to fix a wrong speed report to SNMP.

- Add the "unattended" installation option in the INF file for Windows NT driver

(miniport).

- Modify strings for board identification and connection type.

- Add a new keyword to disable the Link check status before the

send routine for WIN95, preventing the NO_CABLE_CONNECTION

status report to the wrapper (for miniport driver only).

- Support workaround for potential hang of the Rx process in

the 21143-PB (see errata for details). This support was added

in version V4.02.

- Reset the BufferInUse variables at software reset time.

- Fix a leakage in the Free Map Registers pool (in NDIS4.0 only).

The leakage was caused by a wrong exit from the "send" routine when

the retransmision of a packet (with previous underrun) succeeded.

Release V4.16:

- Fix a no LinkStatus indication when the DC21143 card is connected through the 10Base2

port at power-up time.

Release V4.15:

- Fix link interrupt masking on 100Mbps force mode after software reset.

- Fix an Autonegotiation timing problem between 21143/QSI based adapter in one

machine and 21x4/National in the other, after software reset.

- Fix SROM V4.0 detection.

- Disable the HEARTBEAT counter in 21143/QSI based adapters.

Release V4.14:

- This release kit contains two drivers that use Microsoft's NDIS4.0

wrapper for Windows NT 4.0 and the OEM Service Release 2 (OSR2) of

Windows 95.  These drivers are located in subdirectories named

\WNT40\NDIS4.0 and \W95_osr2\NDIS4.0.  Although these drivers are fully

functional, they should not be used for Microsoft certification.  We

recommend that certification be performed using the drivers that use

Microsoft's NDIS3 wrapper.  For Windows 95, use the driver in \WIN95

or W95_osr2\NDIS31 and for Windows NT 4.0 use the driver in \WNT40\NDIS31.

- Fix a Nway problem on the old 21142/ML6692 cardubs design

- Fix Full duplex issue which caused failures on test suite

and performance drop.

- Mask interrupts on different Link statuses to improve performance.

Release V4.13:

- Final release (full NWAY support) DC21143-ML6692 board.

- Support Unlimited Burst Length option.

Release V4.12:

- Fix a DC21143-BroadcomT4 problem.

Release V4.11:

- Support Magic Packet SROM fromat.

Release V4.1:

- Final release for the 21143+QSI 6611 boards.

Release X4.1-004:

- Fixed a problem in force AUI.

- Fixed a problem in force 100-FDX with 21143+QSI

Note: The 21143+Microlinear MII Phy, works OK in the modes

10BaseT Force, 100BaseTx Force, and in NWAY mode succeeds

to do parallel detection to 10BaseT and 100BaseTx media.

The NWAY protocol doesn't work.

The 21143+QSI Phy works well in all the mediums, although

a number of LinkChange interrupts were seen in some revisions

of the QSI Phy.

Release X4.1-003:

- Add support for the DC21143 controller.

- Add support for PCI Memory_Read_Multiple command

- Fix the Adapter's deallocation resource routine

- Fix the No_Carrier and Loss_of_Carrier bogus statuses

for MII + Full_Duplex mode

- Add dynamic Txm Threshold algorithm

Release 4.03:

- Modified detection of MII phy to work on boards

where Phy Addr pins are not pulled up/down correctly.

Release 4.02:

- Includes the Overflow fix for DC21140 and DC21142

- Add the Registry key InterruptMitigation to enable

the interrupt mitigation algorithm

- Fix the oemsetup.inf file for DC21142

Release 4.01:

- First NDIS4.0 driver release

- New handling of the receive buffers and

receive packets

- Full_duplex spinlocks

- NdisIndicateStatus on link transitions

- Dynamic Line_speed and MAC_OPTIONS OIDs,

new OID_GEN_MEDIA_CONNECT_STATUS

- Add support for the Microlinear ML6692 PHY adapter.

- Add speed sensing workaround for National's

DP8340 PHY adapter.

- Modify the handling of Mii register's read failure

in the MiiPhyRegister routine.

- Add the dynamic setting of StoreAndForward transmit

mode if the number of underruns reachs the

Underrun threshold (default=10, programmable through the

Registry's key "UnderrunThreshold").

- Modify the support for Nway media autonegotiation

for the DC21041 Rev2.0 and DC21142 Rev 1.x.

- Add support for the DC21142 controller.

- Store the driver's revision information into

Windows95' Registry and WfW's protocol.ini file:

WNT's Registry:

HKEY_LOCAL_MACHINE\

Software\Intel\DC21X4\CurrentVersion

MajorVersion

MinorVersion

Win95's Registry:

HKEY_LOCAL_MACHINE\

System\CurrentControlSet\Services\Class\Net\"num"\NDIS

MajorDriverVersion

MinorDriverVersion

WfW's protocol.ini :

[DEC$DC21xxx]

MajorDriverVersion

MinorDriverVersion

Release 3.1:

- Single media adapter with no link indication supported

by the Autosense option

- Remove the pageable_function attribute of the 'DC21X4MediaDetect' and

'DC2114Sense100BaseTxLink' routines to allow their run_time invocations

by the Reset handler.

Release 3.0:

- Add Nway support for DC21041

- Separate transmit threshold values for 10Base and 100Base media

Both transmit threshold  values are programmable through the Registry:

- "TxmThreshold" for 10base media

- "TxmThreshold100" for 100base media (meaningless in

StoreAndForward mode)

- Fix unaligned pointers

- Add MII management support for MII's 100baseTx and

100baseT4 PHY adapters

- Updated Registry ConnectionType:

+-----+------------------+------------------------------------------+

|(dec)| Connection Type  |                                          |

+-----+------------------+------------------------------------------+

|  0  | AutoSense        |  AutoSense                               |

+-----+------------------+------------------------------------------+

|  1  | 10Base2          |  BNC,Thinwire,Coax                       |

+-----+------------------+------------------------------------------+

|  2  | 10BaseT          |  Twisted_Pair Half_Duplex                |

+-----+------------------+------------------------------------------+

|  3  | 10BaseT,FD       |  Twisted_Pair Full_Duplex                |

+-----+------------------+------------------------------------------+

|  4  | 10BaseT,NLT      |  Twisted_Pair Half_Duplex,No_Link_Test   |

+-----+------------------+------------------------------------------+

|  5  | 10Base5          |  AUI,Thickwire                           |

+-----+------------------+------------------------------------------+

|  6  | AutoSense,No_Nway|  AutoSense,Nway Disabled                 |

+-----+------------------+------------------------------------------+

|  7  |     -            |  Reserved                                |

+-----+------------------+------------------------------------------+

|  8  | 100BaseTx        |  100BaseTx(Symbol_Scrambler),Half_Duplex |

+-----+------------------+------------------------------------------+

|  9  | 100BaseTx,FD     |  100BaseTx(Symbol_Scrambler),Full_Duplex |

+-----+------------------+------------------------------------------+

| 10  | 100BaseT4        |  100BaseT4                               |

+-----+------------------+------------------------------------------+

Miniport driver only:

- Add Data copy to preallocated buffer instead of

Physical Mapping for Transmit Packets smaller or equal

to 256 bytes.

- Add Interrupt mitigation algorithm:

Interrupt/sec and Frames/sec threshold are programmable

through the Registry's keys "InterruptThreshold" & "FrameThreshold"

- Underrun retransmission. In case of underrun packets are

automaticaly retransmitted by the driver. The maximun number of

retransmission is programmable through the Registry's key

"UnderrunRetry".

Release 2.44:

- Remove BurstLength=32 option for DCc1040 & Dc21041

Release 2.43:

- Add PowerPC miniport

Release 2.42:

- Fix the PCI Mechanism 1 detection for (OPTI platforms) in the

WfW driver

- Fix the SROM parsing for 10BaseT Full Duplex mode

- Fix the DC21040's autodetect hang when the adapter is

unconnected

- Fix the DC2141's Autosense routine

Release 2.41:

- Enhance the DC21140 AutoDetection and AutoSense

routines.

- Add support for DC21140's 100BaseT4 and

100BaseFx media.

- Add support for PCI to PCI bridge in Windows for Workgroups

driver

Release 2.31:

- Number of map registers allocated by the adapter

configurable through the Registry

- Add missing NdisMCancelTimer in the Halt handler

- Add support for DE500 Fast Ethernet adapter

- Remove DE436 from the adapter list

- Add support for ZYNX 21040 boards rev. 03B and

priors

Release 2.3:

- Fix DC21040 and DC21041's Full Duplex mode

Release 2.2:

- Fix the hash filter generation

- Add optionnal Txm FIFO Threshold values

for 100Mbps in the Registry

- Miniport driver fixes for WIN95

- Add Snooze mode option for Intel's 21041

Release 2.1:

- Modify the 10/100 link autosensing algorithm to avoid

switching media while autosensing for link pass:

With the new algorithm the media is switched only if:

- the selected media is 10 Mbps and the 100Mbps link passes:

media is switched from 10Mbps to 100Mbps.

- the selected media is 100Mbps. Its link fails but

the 10 Mbps link passes: the media is switched from

100 Mbps to 10Mbps.

- Modify the ConnectionType values in the driver

installation file to be backward compatible with the DC21040

V1.x values:

Release 2.0:

- Miniport driver for WNT

- Add support for Serial ROM format V1

- Add support for Intel's 21041

- Add dynamic autosense for Intel's 210140 & 21041

- Increase the minimal wait time for BNC

transceiver's setup to 300 milliseconds

- Fix Promiscuous filter setting

- Add support for EB140 and EB41 evaluation boards

- Poll Demand workaround for Motorola's Eagle pass1 chip set

- Support for on_board DC21040 on MR4000 platform

Release 1.2:

- Added workaround for Intel Host to PCI bridge

in WfW driver

- TransceiverDelay in 100Millisecond units

Release 1.1:

- Enhanced interrupt service routine.

- Support Intel's 21140 requirements for size of Multicast address table.

- Fix loopback routine

Windows NT:

- Support DE434 (PCI Ethernet Adapter) and DE436 (PCI Quad Ethernet

Adapter)

Windows for Workgroups:

- Support DE434 (PCI Ethernet Adapter).

Windows 95:

- First release

Release 1.0:

- Added support for Intel's 21140 Fast Ethernet Controller.

- Added static keyword to OEMSETUP.INF: AdapterCFID

- Change driver name to DC21X4.

Copyright Notice:

**************************************************************************

**                                                                      **

**                  I N T E L   P R O P R I E T A R Y                   **

**                                                                      **

**     COPYRIGHT (c)  1998 BY  INTEL  CORPORATION.  ALL RIGHTS          **

**     RESERVED.   NO  PART  OF THIS PROGRAM  OR  PUBLICATION  MAY      **

**     BE  REPRODUCED,   TRANSMITTED,   TRANSCRIBED,   STORED  IN  A    **

**     RETRIEVAL SYSTEM, OR TRANSLATED INTO ANY LANGUAGE OR COMPUTER    **

**     LANGUAGE IN ANY FORM OR BY ANY MEANS, ELECTRONIC, MECHANICAL,    **

**     MAGNETIC,  OPTICAL,  CHEMICAL, MANUAL, OR OTHERWISE,  WITHOUT    **

**     THE PRIOR WRITTEN PERMISSION OF :                                **

**                                                                      **

**                        INTEL  CORPORATION                            **

**                                                                      **

**                     2200 MISSION COLLEGE BLVD                        **

**                                                                      **

**               SANTA  CLARA,  CALIFORNIA  95052-8119                  **

**                                                                      **

**************************************************************************

Copyright (c) 1998 Intel Corporation

Windows and Windows NT are trademarks of

Microsoft Corporation.

Microsoft, MS, MS-DOS are registered trademarks of

Microsoft Corporation.

Magic Packet is a trademark of Advanced Micro Devices

All other trademarks and registered trademarks are the property

of their respective holders.

Important Notice

As of May 17, 1998, Digital Equipment Corporation's StrongARM,

PCI Bridge, and Networking component businesses, along with the chip

fabrication facility in Hudson, Massachusetts, were acquired by

Intel Corporation and transferred to Intel Massachusetts, Inc.

As a result of this transaction, certain references to web sites,

telephone numbers, and fax numbers have changed in the documentation.

This information will be updated in the next version of this manual.

Copies of documents that have an ordering number and are referenced

in this document, or other Intel literature may be obtained by calling

1-800-332-2717 or by visiting Intel's website for developers at:

http://developer.intel.com

The Intel Massachusetts Customer Technology Center continues to

service Network Product technical inquiries. Please use the

following information lines for support:

For documentation and general information:

Intel Massachusetts Information Line

United States: 1-800-332-2717

Outside United States: +1-303-675-2148

Electronic mail address: techdoc@intel.com

For technical support:

Intel Massachusetts Customer Technology Center

Phone (U.S. and international): 1-978-568-7474

Fax: 1-978-568-6698

Electronic mail address: techsup@intel.com

Upgrading Drivers

See the components page available on the Intel World Wide Web

Internet site at:

http://developer.intel.com/design/network/new21/download/dsc-software-nc.htm